# 1049 WASHINGTON AVENUE

# BRONX, NEW YORK

# Remedial Action Work Plan

NYC VCP Project Number: 24CVCP003X

OER Project Number: 23EHAZ-191X

Prepared For:

W. Residence LLC

185-08 Union Turnpike, Suite 106

Fresh Meadows, NY  11366

(718) 801-3431

Prepared By:

Rock Enviro LLC

170 Lee Avenue

Brooklyn NY

(718) 858-66565

JULY 2023

REMEDIAL ACTION WORK PLAN

TABLE OF CONTENTS

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

FFIGURES	iii

APPENDICES	iii

LIST OF ACRONYMS	iv

CERTIFICATION	vi

EXECUTIVE SUMMARY	1

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

REMEDIAL ACTION WORK PLAN	18

1.0  	Project Background	18

1.1  	Site Location and Background	18

1.2   	Redevelopment Plan	18

1.3   	Description of Surrounding Property	19

1.4   	Summary of Past Site Uses and Areas of Concern	20

1.5   	Summary of Work Performed under the Remedial Investigation	21

1.6   	Summary of Findings of Remedial Investigation	22

2.0   	Remedial Action Objectives	25

3.0    	Remedial Alternatives Analysis	26

3.1   	Threshold Criteria Protection of Public Health and the Environment	28

4.0   	Remedial Action	37

4.1   	Summary of Preferred Remedial Action	37

4.2   	Soil Cleanup Objectives and Soil/ Fill Management	40

Error! Hyperlink reference not valid.

4.3   	Engineering Controls	44

4.4   	Institutional Controls	46

4.5   	Site Management Plan	47

4.6   	Qualitative Human Health Exposure Assessment	48

Error! Hyperlink reference not valid.

5.0   	Remedial Action Management	52

5.1   	Project Organization and Oversight	52

5.2   	Site Security	53

5.3   	Work Hours	53

5.4   	Construction Health and Safety Plan	53

5.5   	Community Air Monitoring Plan	54

5.6   	Agency Approvals	56

5.7   	Site Preparation	56

5.8     	Traffic Control	60

5.9     	Demobilization	60

5.10    	Reporting and Record Keeping	60

5.11    	Complaint Management	61

5.12    	Deviations from The Remedial Action Work Plan	61

6.0	Remedial Action Report	62

7.0	Schedule	65

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

Error! Hyperlink reference not valid.

# FIGURES

Figure 1: Site Location Map

Figure 2: Site Boundary Map

Figure 3: Surrounding Land Usage

Figure 4: Map of Endpoint Sample Locations

Figure 5: Site Excavation Diagram

Figure 6: Site-wide Composite Cover System Layout

Figure 7: Composite Cover System Detail

Figure 8: Vapor Barrier System Layout

Figure 9: Vapor Barrier System Detail / Cross-Section

Figure 10: Passive Sub-Slab Depressurization System Layout

Figure 11: Passive Sub-Slab Depressurization System Riser Diagram

Figure 12: Backfill/Import Locations of Bluestone for SSDS System

Figure 13: Map Showing Truck Route

# APPENDICES

Appendix 1: Proposed Redevelopment Plans

Appendix 2: Citizen Participation Plan (CPP)

Appendix 3: Sustainability Statement

Appendix 4: Soil/Materials Management Plan (SMMP)

Appendix 5: Construction Health and Safety Plan (CHASP)

Appendix 6: Vapor Barrier Manufacturer Specifications

Appendix 7: Passive Sub-Slab Venting System Specifications

# LIST OF ACRONYMS

| Acronym | Definition |
| --- | --- |
| AOC | Area of Concern |
| AS/SVE | Air Sparging/Soil Vapor Extraction |
| BOA | Brownfield Opportunity Area |
| CAMP | Community Air Monitoring Plan |
| C&D | Construction and Demolition |
| CEQR | City Environmental Quality Review |
| CFR | Code of Federal Regulations |
| CHASP | Construction Health and Safety Plan |
| COC | Certificate of Completion |
| CQAP | Construction Quality Assurance Plan |
| CSOP | Contractors Site Operation Plan |
| DCR | Declaration of Covenants and Restrictions |
| ECs/ICs | Engineering Controls and Institutional Controls |
| ELAP | Environmental Laboratory Accreditation Program |
| HASP | Health and Safety Plan |
| HAZWOPER | Hazardous Waste Operations Emergency Response |
| IRM | Interim Remedial Measure |
| MNA | Monitored Natural Attenuation |
| NOC | Notice of Completion |
| NYS DEC | New York State Department of Environmental Conservation |
| NYC DEP | New York City Department of Environmental Protection |
| NYC DOHMH | New York State Department of Health and Mental Hygiene |
| NYC OER | New York City Office of Environmental Remediation |
| NYC VCP | New York City Voluntary Cleanup Program |
| NYCRR | New York Codes Rules and Regulations |
| NYS DEC | New York State Department of Environmental Conservation |
| NYS DEC DER | New York State Department of Environmental Conservation Division of Environmental Remediation |
| NYS DOH | New York State Department of Health |
| NYS DOT | New York State Department of Transportation |
| ORC | Oxygen-Release Compound |
| OSHA | United States Occupational Health and Safety Administration |
| PCBs | Polychlorinated Biphenyls |
| PE | Professional Engineer |
| PID | Photo Ionization Detector |
| QEP | Qualified Environmental Professional |
| QHHEA | Qualitative Human Health Exposure Assessment |
| RAOs | Remedial Action Objectives |
| RAR | Remedial Action Report |
| RAWP | Remedial Action Work Plan or Plan |
| RCA | Recycled Concrete Aggregate |
| RD | Remedial Design |
| RI | Remedial Investigation |
| RMZ | Residual Management Zone |
| SCOs | Soil Cleanup Objectives |
| SCG | Standards, Criteria and Guidance |
| SMP | Site Management Plan |
| SPDES | State Pollutant Discharge Elimination System |
| SSDS | Sub-Slab Depressurization System |
| SVOC | Semi-Volatile Organic Compound |
| TAL | Target Analyte List |
| TCL | Target Compound List |
| USGS | United States Geological Survey |
| UST | Underground Storage Tank |
| VCA | Voluntary Cleanup Agreement |
| VOC | Volatile Organic Compound |

# CERTIFICATION

I, [name], am currently a registered professional engineer licensed by the State of New York. I performed professional engineering services and had primary direct responsibility for designing the remedial program for 1049 Washington Avenue, Bronx NY, Site number 23EHAZ191X/24CVCP003X.  I certify under the appropriate standard of care to the following:

- I have reviewed this document and the Stipulation List, to which my signature and seal are affixed.

- Engineering Controls developed for this remedial action were designed by me or a person under my direct supervision and designed to achieve the goals established in this Remedial Action Work Plan for this site.

- The Engineering Controls to be constructed during this remedial action are accurately reflected in the text and drawings of the Remedial Action Work Plan and are of sufficient detail to enable proper construction.

- This Remedial Action Work Plan (RAWP) has a plan for handling, transporting, and disposing of remediation-related soil, fill and fluids removed from the property in accordance with applicable City, State, and Federal laws and regulations. The RAWP also has a plan for the importation of all soil, fill and other material from off-Site in accordance with all applicable City, State and Federal laws and requirements. This RAWP has provisions to control nuisances during the remediation and all invasive work, including dust and odor suppression.

_________

PE Stamp

Name

PE License Number

Signature

Date

I, Robert M. Bennett, am a qualified Environmental Professional.  I will have primary direct responsibility for the implementation of the remedial program for 1049 Washington Avenue, Bronx NY, site number 23EHAZ191X / 24CVCP003X. I certify under the appropriate standard of care to the following:

- This Remedial Action Work Plan (RAWP) has a plan for handling, transporting, and disposing of remediation-related soil, fill and fluids removed from the property in accordance with applicable City, State and Federal laws and regulations.  The RAWP also has a plan the or importation of all soil, fill and other material from off-Site in accordance with all applicable City ,State and Federal laws and requirements. This RAWP has provisions to control nuisances during the remediation and all invasive work, including dust and odor suppression.

Robert M. Bennett

QEP Name

QEP Signature

July 19, 2023

Date

# EXECUTIVE SUMMARY

W. Residence LLC is working with the NYC Office of Environmental Remediation (OER) in the New York City Voluntary Cleanup Program to investigate and remediate a 27,132-square foot (SF) property located at 1049 Washington Avenue in Bronx, New York. A remedial investigation (RI) was performed to compile and evaluate the data and information necessary to develop this Remedial Action Work Plan (RAWP). The remedial action described in this document protects public health and the environment consistent with the intended property use, complies with applicable environmental standards, criteria and guidance and conforms with applicable laws and regulations.

# Site Location and Background

# The Site is located at 1049 Washington Avenue in the Morrisania section of Bronx, New York, and is identified as Block 2387 and Lots 44, 49, and 51 on the New York City Tax Map.  Figures 1 and 2 show the Topographic Map and the Site Location, respectively.  The Site is 27,132-square feet and is bounded by mixed-use commercial and residential buildings to the north, followed by East 166th Street to the north; immediately bounded by East 165th Street to the south, followed by mixed use commercial and residential buildings; immediately bounded by Washington Avenue to the east, followed by mixed-use commercial and residential buildings; and mixed-use commercial and residential buildings, followed by Park Avenue to the west. A map of the site boundary is shown in Figure 3.  Currently, the Site is vacant and contains a one-story commercial warehouse with a partial cellar and associated parking lots.

# Summary of Redevelopment Plan

The development project consists of a new 11-story mixed-use commercial and residential building fronting Washington Avenue. The proposed building will consist of a full build-out where the cellar level will be utilized for automobile parking of approximately 64 parking spaces, bicycle storage, building services, laundry, and a trash compactor room; the first floor will consist of commercial space, security services, lobby, a mailbox room, and parking access from East 165th Street; the second and third floors will be utilized as a day care center, with additional automobile parking (approximately 52 parking spaces) on the second floor; and the remaining eight floors (fourth through 11th) will be dedicated to approximately 104 residential units. The proposed building will feature terraces for building maintenance on the second and third floors located at the rear of the property, and tenant terraces on the fourth floor, set back approximately 15 feet from Washington Avenue and East 165th Street, with another terrace in the rear of the property.

As part of development, the referenced lots are expected to be merged into Lot 44. The proposed depth of excavation is approximately 10 feet below grade surface (ft bgs) for construction of the cellar, and approximately 15 ft bgs for the elevator pits. The water table is expected to be at approximately 19 feet bgs, based on the Remedial investigation performed by Rock Enviro in April 2023, therefore dewatering is not anticipated to occur during remediation/construction at the Site. An estimated 11,000 cubic yards (16,500 tons) of soil will be removed for redevelopment purposes.

The layout of the proposed site development is presented in Figure 4. The current zoning designations are mixed-use, manufacturing, and residential zones M1-1/R7-2. The proposed use is consistent with existing zoning for the property.

# Summary of Surrounding Property

The general vicinity of the surrounding area comprises of residential and commercial properties. The site is bounded by mixed use commercial and residential buildings to the north; immediately bounded by East 165th Street followed by mixed use commercial and residential buildings to the south; immediately bounded by Washington Avenue followed by mixed-use commercial and residential buildings, including Sunshine Devel-Sharon Baptist HS, Sharon Baptist Head Start, and little Scholars Early Development Center, LLC to the east; and mixed-use commercial and residential buildings to the west. No other sensitive receptors such as schools, hospitals, or day care facilities are located within 500 feet of the site.

# Summary of Past Site Uses and Areas of Concern

Phase I Environmental Site Assessment (ESA) was completed by Rock Enviro LLC (Rock). Based on the Sanborn Maps and City Directory Listing, the following site history was established:

- In 1891, the site was occupied by a church and nine unspecified buildings. In 1951 the site was occupied by a private garage, bakery, story, and dwelling with the addition of a parking lot in 1969.

- In 1970-1984, the site is occupied with manufacturing.

- In 1989 the site is occupied by auto repair and vacant land.

- In 1991 to 2007, the site is occupied by Ambassador Fuel and Oil Burner Corp, and vacant land.

Prior site ownership based on review of the New York City Department of Finance Records Lists the following:

| Name of Property Owner | Purchase year |
| --- | --- |
| Lot 44 |  |
| 1976-1983 | Welbilt Electronic Die Corp |
| 1983-1988 | Vanora, LTD, Wedtech Corp, Garard Note Corp |
| 1989-2019 | A&D Realty Company |
| 2019-2022 | Hello Washington Corp, Psalms 94 LLC, 1000 Claremont Village LLC |
| Name of Property Owner | Purchase year |
| Lot 49 |  |
| 1966 | Private residence |
| 2019-2022 | Hello Washington Corp, Psalms 94 LLC, 1000 Claremont Village LLC |
| Name of Property Owner | Purchase year |
| Lot 51 |  |
| 1976 | Geejay Realty Corp |
| 1977 | Primm Howered |
| 1991-2019 | Bilotti Florr/Ex, 165 Associates |
| 2019-2022 | Hello Washington Corp, Psalms 94 LLC, 1000 Claremont Village LLC |

Recognized environmental conditions (RECs) and areas of concern (AOCs) in association with the Site were identified in the Phase I ESA completed by Rock. The following was identified for the subject property.

- A review of the Municipal Records and Sanborn maps indicated that the subject site, 1049 Washington Avenue (Lot 44), contained two gasoline underground storage tanks (USTs) from 1951-1970. In addition, the subject Site, 1049 Washington Avenue (Lot 44), was improved with an unspecified manufacturer from 1970-1984, and an auto repair facility in 1989, and lastly developed with Ambassador Fuel & Oil Burner Corp. from 1991-2007.

- A review of a 2018 Phase II Subsurface Investigation Report for the subject site prepared by Environmental Business Consultants (EBC) indicated that PCE was detected at low but elevated concentrations in one of the two groundwater samples collected.

- A review of EDR Database indicated nearby property, 1029 Washington Avenue, is at a higher elevation, cross gradient, and 164 ft southeast of the subject sites. This property is identified in the EDR Database as a historic auto, reported spill incident, and has been assigned E-Designation, E-118, for underground gasoline storage tanks testing protocol by the New York City Department of City Planning as part of the Morrisania rezoning action (CEQR # 03DCP046X). Further review indicates three groundwater wells were installed at the adjacent property and gasoline contamination was reported in a single well adjacent to the gas station located at 1029 Washington Avenue.

- A review of the Municipal Records indicated that Lot 51, located at 439 East 165th Street, has been assigned E-Designation, E-118, for underground gasoline storage tanks testing protocol as part of the Morrisania rezoning action (CEQR #03DCP046X).

# Summary of Work Performed under the Remedial Investigation

Rock performed the following scope of work:

- Conducted a Site inspection to identify AOCs and physical obstructions (i.e., structures, buildings, etc.).

- Installed nine (9) soil borings across the entire project Site and collected twenty (20) soil samples for chemical analysis from the soil borings to evaluate soil quality.

- Installed five (5) groundwater monitoring wells throughout the Site to establish groundwater flow and collected six (6) groundwater samples for chemical analysis to evaluate groundwater quality; and,

- Installed eight (8) soil vapor probes around Site perimeter and collected eight (8) samples for chemical analysis.

# Summary of Findings of Remedial Investigation

- The elevation of the property ranges from 26.00 to 28.42 feet North American Vertical Datum of 1988 (NAVD 88).

- Depth to groundwater ranges from 17.2 to 18.8 feet at the Site.

- Groundwater flows from west to east beneath the Site.

- Depth to bedrock is unknown. Bedrock was not encountered during this RI. Based on regional information available to Rock, and a Geotechnical Investigation performed at the Site in February 2022, bedrock in the surrounding areas is anticipated at depths greater than 40 feet below grade.

- The stratigraphy of the site, from the surface down, consists of 8 feet of fill material, comprising of debris (trash, brick, broken rock and concrete) with fine to medium sand, underlain by red brown to tan medium to find grained sand with coarse sand, granules, and crushed rock throughout to a maximum depth of 17 ft bgs.

- Soil/fill samples collected during the RI were compared to 6 NYCRR NYSDEC Part 375.6-8(a) Unrestricted Use Soil Clean Up Objectives (SCOs), Residential, Restricted Residential, and Commercial Use SCOs.

- No VOCs, PCBs, or emerging contaminants were detected at concentrations above their respective Unrestricted Use SCOs.

- Five (5) SVOCs including, benzo(a)anthracene (max concentration of 11 mg/Kg), benzo(a)pyrene (max concentration of 11.7 mg/Kg), benzo(b)fluoranthene (max concentration of 9.52 mg/Kg), dibenzo (a, h) anthracene (max concentration of 2.02 mg/Kg), and indeno(1,2,3-cd)pyrene (max concentration of 7.59 mg/Kg), were detected at concentrations exceeding their Commercial Use SCO. Two (2) SVOCs including benzo(k)fluoranthene (max concentration of 8.65 mg/Kg) and chrysene (max concentration of 10.8 mg/Kg), were detected at concentrations exceeding their Restricted Residential Use SCOs. All maximum exceedances were located within SB-7 (0-2). SVOCs in all deeper soil borings (10-12’) were detected at trace concentrations, below Unrestricted Use SCOs.

- Three (3) pesticides including 4-4’-DDD (max concentration of 0.00361 mg/Kg in DUP 2), 4,4’-DDE (max concentration of 0.00669 mg/Kg in SB-2 0-2), and 4,4’ DDT (max concentration of 0.0458 mg/Kg in DUP 2), were detected at concentrations exceeding their Unrestricted Use SCOs.

- Four (4) metals, including barium (max concentration 906 mg/Kg in SB-2 0-2), copper (max concentration of 416 mg/Kg in SB-7 0-2), lead (max concentration of 1,160 mg/Kg in Dup 2), and Nickel (max concentration of 1,230 m/Kg in SB-5 10-12), were detected at concentrations exceeding their Commercial Use SCOs. One (1) metal, mercury (max concentration of 1.28 mg/Kg in SB-6 0-2) was detected at a concentration above its Restricted Residential Use SCO. One (1) metal, cadmium (max concentration of 3.75 mg/Kg in SB-6 0-2) was detected at a concentration above its Residential Use SCO. Three (3) metals including, arsenic (max concentration of 15.5 mg/Kg in SB-6 0-2), selenium (max concentration of 6.22 mg/Kg in SB-1 0-2), and zinc (max concentration of 875 mg/Kg in SB-6 0-2), were detected at concentrations exceeding their Unrestricted Use SCOs.

- Groundwater samples collected during the RI were compared to the NYSDEC 6NYCRR Part 703.5 Class GA groundwater quality standards (GQS).

- No PCBs or 1,4 dioxane were detected in any of the groundwater samples at concentrations above their GQS.

- Two (2) VOCs, including PCE (max. of 10.8 ug/L in GW 1), and trichloroethylene (TCE) (max of 6.33 ug/L in GW-DUP), were detected at concentrations above their GQS.

- Six (6) SVOCs, including benzo(a)anthracene (max. concentration of 0.350 ug/L), benzo(a)pyrene (max. concentration of 0.380 ug/L in GW-2), benzo(b)fluoranthene (max concentration of 0.340 ug/L), benzo(k)fluoranthene (max concentration of 0.330 ug/L), bis(2-ethylhexl)phthalate (max concentration of 13.7 ug/L), chrysene (max concentration of 0.41 ug/L),indeno (1,2,3-cd)pyrene (max concentration of 0.23 ug/L), and naphthalene (max concentration of 10.1 ug/L) were detected at concentrations above their GQS. All maximum exceedances were detected within GW-2.

- One (1) pesticide, dieldrin (max concentration of 0.00449 g/L in GW-2), was detected at a concentration above the GQS.

- Four (4) metals including lead (max concentration of 62 ug/L in MW-2), magnesium (max concentration 69,100 ug/L in GW-4), manganese (max concentration 943 ug/L in GW-5), selenium (max concentration 74 ug/L in GW-3), and sodium (max concentration of 80,700 ug/L in GW-2), were detected at concentrations above their GQS.

- Two (2) dissolved metals, selenium (max concentration of 74.6 ug/L) and sodium (max concentration of 80,300 ug/L), were detected at concentrations above their GQS. All maximum exceedances were detected within GW-3.

- Two (2) emerging contaminates, including Perfluorooctanesulfonic acid (PFOS) (max concentration of 91.3 ug/L in GW-1) and Perfluorooctanoic acid (PFOA) (max concentration of 25.1 ug/L in GW-3), were detected at concentrations above their GQS.

- Soil vapor samples collected during the RI were compared to the compounds listed in table 3.1 of the Air Guideline Values derived by the NYSDOH located in New York State Department of Health (NYSDOH) Final Guidance for Evaluating Soil Vapor Intrusion, dated October 2006, and the revised NYSDOH Division Matrices, dated May 2017.

- Soil vapor results indicate elevated levels of chlorinated VOCs (CVOCs).

- Total concentrations of petroleum-related VOCs, benzene, toluene, ethylbenzene, and xylenes (BTEX) ranged from 254.1 micrograms per cubic meter (ug/m3) to 885 ug/m3, with the maximum concentration located in SV-6.

- CVOC concentrations include 1,1,1-Trichloroethane max concentration of 76 ug/m3), (cis-1,2-Dichloroethylene (max concentration of 12 ug/m3), PCE (max concentration of 1,300 ug/m3) and TCE (max concentration of 280 ug/m3) with maximum concentrations detected in SV-1. Methylene chloride (max concentration of 42 ug/m3 in SV-3) was also detected in the soil vapor samples.  Maximum concentrations of TCA, TCE, and PCE were detected in single soil vapor; location SV-1.

- 1,1-Dichloroethylene (max concentration of 0.67 ug/m3) and carbon tetrachloride (max concentration of 1.3 ug/m3) were detected in SV-8 only.

- Vinyl chloride was not detected in any of the soil vapor samples collected from the site.

# Summary of the Remedial Action

The proposed remedial action achieves the protection of public health and the environment for the intended use of the property. The proposed remedial action achieves all of the remedial action objectives established for the project and addresses applicable standards, criterion, and guidance; is effective in both the short-term and long-term and reduces mobility, toxicity and volume of contaminants; is cost-effective and implementable; and uses standards methods that are well established in the industry.

The proposed remedial action will consist of:

    - Preparation of a Community Protection Statement and performance of all required NYC VCP Citizen Participation activities according to an approved Citizen Participation Plan.

    - Performance of a Community Air Monitoring Program for particulates and volatile organic carbon compounds;

    - Establishment of Track 4 Site-Specific Soil Cleanup Objectives (SCOs). Track 4 Site-Specific SCOs selected for this project are SVOCs at 100 ppm, Lead at 800 ppm, Copper at 270 ppm, Mercury at 1.5 ppm, Barium at 650ppm and protection of groundwater standards;

    - Site mobilization involving Site security setup, equipment mobilization, utility mark outs, and marking & staking excavation areas;

    - Completion of a Waste Characterization Study prior to excavation activities. Waste characterization soil samples will be collected at a frequency dictated by disposal facility. A Waste Characterization Report, documenting sample procedures, location, and analytical results shall be submitted to NYCOER prior to the start of remedial action;

    - Excavation and removal of soil/fill exceeding Track 4 Site-Specific SCOs. The entire site will be excavated to a depth of 10 feet below grade surface (bgs) for the proposed building cellar. The elevator pits will be excavated to a maximum depth of 15 feet bgs. An estimated 11,000 cubic yards (16,500 tons) of soil/fill will be removed from the Site and properly disposed of at an appropriately licensed or permitted facility;

    - Screening of excavated soil/fill during intrusive work for indications of contamination by visual means, odor, and monitoring with a PID;

    - Management of excavated materials including temporarily stockpiling and segregating in accordance with defined material types and to prevent co-mingling of contaminated material and non-contaminated materials;

    - Removal of all underground storage tanks (UST’s) encountered during soil/fill removal actions. Registration of tanks and reporting of any petroleum spills associated with UST’s and appropriate closure of these petroleum spills in compliance with applicable local, State and Federal laws and regulations;

    - Transportation and off-Site disposal of all soil/fill material at permitted facilities in accordance with applicable laws and regulations for handling, transport, and disposal, and this plan. Sampling and analysis of excavated media as required by disposal facilities. Appropriate segregation of excavated media on-Site;

    - Collection and analysis of five (5) soil vapor samples after excavation is completed in areas of elevated soil vapor results detected during RIR to evaluate if passive SSDS should be running as active system;

    - Collection and analysis of nine (9) site-wide end-point samples (EP–EP9) across the building footprint to determine the performance of the remedy with respect to attainment of SCOs;

    - Installation of three groundwater wells and collection of groundwater samples for VOCs to further evaluate groundwater quality;

    - Import of materials to be used for backfilling and cover in compliance with this plan and in accordance with applicable laws and regulations;

    - Installation of a passive Sub-Slab Depressurization System (SSDS). The SSDS will consist of six loops installed within porous granular material, ASTM5 ¾ inch bluestone with no fines, beneath the basement foundation. All six loops will be installed within the cellar level of the entire building footprint. The SSDS loops will provide the correct coverage in accordance with USEPA sub-slab depressurization design specifications which recommend a separate vent loop for every 4,000 SF of slab area. The SSDS loops will be outfitted with a cast-iron riser that will extend to the roof of the building. The exhaust will be placed at a minimum distance of 10-15ft from all air intakes. The SSDS is an Engineering Control for the remedial action;

    - Construction of an engineered composite cover consisting of: Cellar Slab; a 6-inch-thick concrete slab underlain with Stego Wrap 20-mil vapor barrier system underlain by an 8-inch layer of ¾” bluestone with SSDS pipe underlain by residual soil;

    - Installation of a vapor barrier system beneath the entire cellar slab, below/around the elevator pita, and behind all cellar walls to grade to mitigate soil vapor migration into the building. The vapor barrier system will consist of Stego Wrap 20-mil vapor barrier system, or OER-approved equivalent system. All welds, seams and penetrations will be properly sealed to prevent preferential pathways for vapor migration;

    - Performance of all activities required for the remedial action, including permitting requirements and pretreatment requirements, in compliance with applicable laws and regulations;

    - Implementation of storm-water pollution prevention measures in compliance with applicable laws and regulations;

    - Submission of a Remedial Action Report (RAR) that describes the remedial activities, certifies that the remedial requirements have been achieved, defines the Site boundaries, lists any changes from this RAWP, and describes all Engineering and Institutional Controls to be implemented at the Site;

    - Submission of an approved Site Management Plan (SMP) in the RAR for long-term management of residual contamination, including plans for operation, maintenance, monitoring, inspection and certification of Engineering and Institutional Controls and reporting at a specified frequency;

    - A deed restriction will be placed on the property to document the installation of, and continued operation, of a passive SSDS. The deed restriction can be removed if OER determines that the active SSDS has achieved its goals and is no longer warranted; and

    - The property will continue to be registered with an E-Designation at the NYC Buildings Department. Establishment of Engineering Controls and Institutional Controls in this RAWP and a requirement that management of these controls will include prohibition of the following: (1) vegetable gardening and farming; (2) use of groundwater without treatment rendering it safe for the intended use; (3) disturbance of residual contaminated material unless it is conducted in accordance with the SMP; and (4) higher level of land usage without OER-approval.

COMMUNITY PROTECTION STATEMENT

The NYC Office of Environmental Remediation (OER) provides governmental oversight for the cleanup of contaminated property in NYC.  This Remedial Action Work Plan (“cleanup plan”) describes the findings of prior environmental studies, shows the location of identified contamination at the site, and describes the plans to clean up the site to protect public health and the environment.

This cleanup plan provides a very high level of protection for neighboring communities and also includes many other elements that address common community concerns, such as community air monitoring, odor, dust and noise controls, hours of operation, good housekeeping and cleanliness, truck management and routing, and opportunities for community participation. The purpose of this Community Protection Statement is to explain these community protection measures in non-technical language to simplify community review.

# Project Information:

- Site Name:  1049 Washington Avenue

- Site Address:  1049 Washington Avenue, Bronx, New York 10456

- OER Project #: 23EHAZ191X / 23TMP1184X

- NYC Voluntary Cleanup Program Project Number:  24CVCP003X

# Project Contacts:

- OER Project Manager: Shaminder Chawla, 212-788-8841

- Site Project Manager:  Robert M. Bennett, 718.858.6655 ext.215

- Site Safety Officer:  Jason Gellati, 718.858.6655 ext. 212

- Online Document Repository:

https://a002-epic.nyc.gov/app/workspace/34544/docrepository

Remedial Investigation and Cleanup Plan:  Under the oversight of the NYC OER, a thorough study of this property (called a remedial investigation) has been performed to identify past property usage, to sample and test soils, groundwater and soil vapor, and to identify contaminant sources present on the property.  The cleanup plan has been designed to address all contaminant sources that have been identified during the study of this property.

Identification of Sensitive Land Uses:  Prior to selecting a cleanup, the neighborhood was evaluated to identify sensitive land uses nearby, such as schools, day care facilities, hospitals and residential areas.  The cleanup program was then tailored to address the special conditions of this community.

Qualitative Human Health Exposure Assessment: An important part of the cleanup planning for the Site is a study to find all of the ways that people might come in contact with contaminants at the Site now or in the future.  This study is called a Qualitative Human Health Exposure Assessment (QHHEA).  A QHHEA was performed for this project.  This assessment has considered all known contamination at the Site and evaluated the potential for people to come in contact with this contamination.  All identified public exposures will be addressed under this cleanup plan.

Health and Safety Plan:  This cleanup plan includes a Construction Health and Safety Plan (CHASP) that is designed to protect community residents and on-Site workers.  The elements of this RAWP are in compliance with applicable safety requirements of the United States Occupational Safety and Health Administration (OSHA).  This RAWP includes many protective elements including those discussed below.

Site Safety Coordinator:  This project has a designated Site safety coordinator to implement the CHASP.  The safety coordinator maintains an emergency contact sheet and protocol for management of emergencies. The Site safety coordinator is identified at the beginning of this Community Protection Statement.

Worker Training:  Workers participating in the cleanup of contaminated material on this project are required to be trained in 40-hour hazardous waste operators training course and to take annual refresher training.  This pertains to workers performing specific tasks including removing contaminated material and installing cleanup systems in contaminated areas.

Community Air Monitoring Plan:  Community air monitoring will be performed during this cleanup project to ensure that the community is properly protected from contaminants, dust and odors.  Air samples will be tested in accordance with a detailed plan called the Community Air Monitoring Plan or CAMP.  Results will be regularly reported to the NYC Office of Environmental Remediation.  This cleanup plan also has a plan to address any unforeseen problems that might occur during the cleanup (called a ‘Contingency Plan’).

Odor, Dust and Noise Control:  This cleanup plan includes actions for odor and dust control.  These actions are designed to prevent off-Site odor and dust nuisances and include steps to be taken if nuisances are detected.  Generally, dust is managed by application of physical covers and by water sprays.  Odors are controlled by limiting the area of open excavations, physical covers, spray foams and by a series of other actions (called operational measures).  The project is also required to comply with applicable NYC noise control standards. If you observe problems in these areas, please contact the onsite Project Manager or NYC Office of Environmental Remediation Project Manager listed on the first page of this Community Protection Statement document.

Quality Assurance:  This cleanup plan requires that evidence be provided to illustrate that all cleanup work required under the plan has been completed properly.  This evidence will be summarized in the final report, called the Remedial Action Report.  This report will be submitted to the NYC Office of Environmental Remediation and will be thoroughly reviewed.

Stormwater Management:  To limit the potential for soil erosion and discharge, this cleanup plan has provisions for stormwater management.  The main elements of stormwater management include physical barriers such as tarp covers and erosion fencing and a program for frequent inspection.

Hours of Operation:  The hours for operation of cleanup will comply with the NYC Department of Buildings construction code requirements or according to specific variances issued by that agency.  For this cleanup project, the hours of operation will conform to requirements of the NYC Department of Buildings.

Signage:  While the cleanup is in progress, a placard will be prominently posted at the main entrance of the property with a laminated project Fact Sheet that states that the project is in the NYC Voluntary Cleanup Program and provides project contact names and numbers, and a link to the document repository where project documents can be viewed.

Complaint Management:  The contractor performing this cleanup is required to address all complaints.  If you have any complaints, you can call the facility Project Manager, or the NYC Office of Environmental Remediation Project Manager listed on the first page of this Community Protection Statement document or call 311 and mention the Site is in the NYC Voluntary Cleanup Program.

Utility Mark-outs:  To promote safety during excavation in this cleanup, the contractor is required to first identify all utilities and must perform all excavation and construction work in compliance with NYC Department of Buildings regulations.

Soil and Liquid Disposal:  All soil and liquid material removed from the Site as part of the cleanup will be transported and disposed of in accordance with all applicable City, State and Federal regulations, and required permits will be obtained.

Soil Chemical Testing and Screening:  All excavations will be supervised by a trained and properly qualified environmental professional. In addition to extensive sampling and chemical testing of soils on the Site, excavated soil will be screened continuously using hand-held instruments, by sight, and by smell to ensure proper material handling and management, and community protection.

Stockpile Management:  Soil stockpiles will be kept covered with tarps to prevent dust, odor and erosion.  Stockpiles will be frequently inspected. Damaged tarp covers will be promptly replaced.  Stockpiles will be protected with silt fences. Hay bales will be used, as needed, to protect storm water catch basins and other discharge points.

Trucks and Covers:  Loaded trucks leaving the Site will be covered in compliance with applicable laws and regulations to prevent dust and odor. Trucks will be properly recorded in logs and records and placarded in compliance with applicable City, State and Federal laws, including those of the New York State Department of Transportation. If loads contain wet material that can leak, truck liners will be used. All transport of materials will be performed by licensed truckers and in compliance with applicable laws and regulations.

Imported Material:  All fill materials proposed to be brought onto the Site will comply with rules outlined in this cleanup plan and will be inspected and approved by a qualified worker located on the Site. Waste materials will not be brought onto the Site. Trucks entering the Site with imported clean materials will be covered in compliance with applicable laws and regulations.

Equipment Decontamination:  All equipment used for cleanup work will be inspected and washed, if needed, before it leaves the Site. Trucks will be cleaned at a truck inspection station on the property before leaving the Site.

Housekeeping:  Locations where trucks enter or leave the Site will be inspected every day and cleaned regularly to ensure that they are free of dirt and other materials from the Site.

Truck Routing:  Truck routes have been selected to: (a) limit transport through residential areas and past sensitive nearby properties; (b) maximize use of city-mapped truck routes; (c) limit total distance to major highways; (d) promote safety in entry to highways; (e) promote overall safety in trucking; and (f) minimize off-Site line-ups (queuing) of trucks entering the property.  Operators of loaded trucks leaving the Site will be instructed not to stop or idle in the local neighborhood.

Final Report:  The results of all cleanup work will be fully documented in a final report (called the Remedial Action Report) that will be available for public review online. A link to the online document repository and the public library with Internet access nearest the Site are listed on the first page of this Community Protection Statement document.

Long-Term Site Management:  If long-term protection is needed after the cleanup is complete, the property owner will be required to comply with an ongoing Site Management Plan that calls for continued inspection of protective controls, such as Site covers. The Site Management Plan is evaluated and approved by the NYC Office of Environmental Remediation. Requirements that the property owner must comply with are defined either in the property’s deed or established through a city environmental designation registered with the Department of Buildings. A certification of continued protectiveness of the cleanup will be required from time to time to show that the approved cleanup is still effective.

# REMEDIAL ACTION WORK PLAN

# 1.0 	Project Background

W. Residence LLC is working with the NYC Office of Environmental Remediation (OER) in the New York City Voluntary Cleanup Program and/or in the “E” Designation Program to investigate and remediate a property located at 1049 Washington Avenue in the Morrisania section of the Bronx, New York (the “Site”).  A Remedial Investigation (RI) was performed to compile and evaluate data and information necessary to develop this Remedial Action Work Plan (RAWP) in a manner that will render the Site protective of public health and the environment consistent with the contemplated end use. This RAWP establishes remedial action objectives, provides a remedial alternatives analysis that includes consideration of a permanent cleanup, and provides a description of the selected remedial action. The remedial action described in this document provides for the protection of public health and the environment, and complies with applicable environmental standards, criteria and guidance and applicable laws and regulations.

# 1.1	Site Location and Background

The Site is located at 1049 Washington Avenue in the Morrisania section in Bronx, New York and is identified as Block 2387 and Lots 44, 49, and 51 on the New York City Tax Map. Figures 1 and 2 show the Topographic Map and the Site Location, respectively. The Site is 27,132-square feet and is bounded by mixed use commercial and residential buildings to the north, followed by East 166th Street to the north; immediately bounded by East 165th Street to the south, followed by mixed use commercial and residential buildings; immediately bounded by Washington Avenue to the east, followed by mixed use commercial and residential buildings; and mixed use commercial and residential buildings, followed by Park Avenue to the west. A map of the site boundary is shown in Figure 3. Currently, the Site is vacant and contains a one-story commercial warehouse with a partial cellar and associated parking lots.

# 1.2	Redevelopment Plan

The development project consists of a new 11-story mixed-use commercial and residential building fronting Washington Avenue. The proposed building will consist of a full build-out where the cellar level will be utilized for automobile parking of approximately 64 parking spaces, bicycle storage, building services, laundry, and a trash compactor room; the first floor will consist of commercial space, security services, lobby, a mailbox room, and parking access from East 165th Street; the second and third floors will be utilized as a day care center, with additional automobile parking (approximately 52 parking spaces) on the second floor; and the remaining eight floors (fourth through 11th) will be dedicated to approximately 104 residential units. The proposed building will feature terraces for building maintenance on the second and third floors located at the rear of the property, and tenant terraces on the fourth floor, set back approximately 15 feet from Washington Avenue and East 165th Street, with another terrace in the rear of the property.

As part of development, the referenced lots are expected to be merged into Lot 44. The proposed depth of excavation is approximately 10 feet below grade surface (ft bgs) for construction of the cellar, and approximately 15 ft bgs for the elevator pit. The water table is expected to be at approximately 19 feet based on the Remedial Investigation performed by Rock Enviro, dated April, 2023. Therefore dewatering is not anticipated to occur during remediation/construction work at the Site. An estimated 11,000 cubic yards (16,500 tons) of soil will be removed for redevelopment purposes.

The layout of the proposed site development is presented in Figure 4. The current zoning designations are mixed use, manufacturing, and residential zones M1-1/R7-2. The proposed use is consistent with existing zoning for the property.

The remedial action contemplated under this RAWP may be implemented independently of the proposed redevelopment plan.

# 1.3	Description of Surrounding Property

The general vicinity of the surrounding area comprises of residential and commercial properties. The site is bounded by mixed use commercial and residential buildings to the north; immediately bounded by East 165th Street followed by mixed use commercial and residential buildings to the south; immediately bounded by Washington Avenue followed by mixed use commercial and residential buildings, including Sunshine Devel-Sharon Baptist HS, Sharon Baptist Head Start, and little Scholars Early Development Center, LLC to the east; and mixed use commercial and residential buildings to the west. No other sensitive receptors such as schools, hospitals, or day care facilities are located within 500 feet of the site.

Figure 3 shows the surrounding land usage.

# 1.4 	Summary of Past Site Uses and Areas of Concern

A Phase I Environmental Site Assessment (ESA) was completed by Rock Enviro LLC (Rock). Based on the Sanborn Maps and City Directory Listing, the following site history was established:

- In 1891, the site was occupied by a church and nine unspecified buildings. In 1951 the site was occupied by a private garage, bakery, story, and dwelling with the addition of a parking lot in 1969.

- In 1970-1984, the site is occupied with manufacturing.

- In 1989 the site is occupied by auto repair and vacant land.

- In 1991 to 2007, the site is occupied by Ambassador Fuel and Oil Burner Corp, and vacant land.

Prior site ownership based on review of the New York City Department of Finance Records Lists the following:

| Name of Property Owner | Purchase year |
| --- | --- |
| Lot 44 |  |
| 1976-1983 | Welbilt Electronic Die Corp |
| 1983-1988 | Vanora, LTD, Wedtech Corp, Garard Note Corp |
| 1989-2019 | A&D Realty Company |
| 2019-2022 | Hello Washington Corp, Psalms 94 LLC, 1000 Claremont Village LLC |
| Name of Property Owner | Purchase year |
| Lot 49 |  |
| 1966 | Private residence |
| 2019-2022 | Hello Washington Corp, Psalms 94 LLC, 1000 Claremont Village LLC |
| Name of Property Owner | Purchase year |
| Lot 51 |  |
| 1976 | Geejay Realty Corp |
| 1977 | Primm Howered |
| 1991-2019 | Bilotti Florr/Ex, 165 Associates |
| 2019-2022 | Hello Washington Corp, Psalms 94 LLC, 1000 Claremont Village LLC |

Recognized environmental conditions (RECs) and areas of concern (AOCs) in association with the site were identified in the Phase I ESA completed by Rock. The following was identified for the subject property.

- A review of the Municipal Records and Sanborn maps indicated that the subject site, 1049 Washington Avenue (Lot 44), contained two gasoline underground storage tanks (USTs) from 1951-1970. In addition, the subject Site, 1049 Washington Avenue (Lot 44), was improved with an unspecified manufacturer from 1970-1984, and an auto repair facility in 1989, and lastly developed with Ambassador Fuel & Oil Burner Corp. from 1991-2007.

- A review of a 2018 Phase II Subsurface Investigation Report for the subject site prepared by Environmental Business Consultants (EBC) indicated that PCE was detected at low but elevated concentrations in one of the two groundwater samples collected.

- A review of EDR Database indicated nearby property, 1029 Washington Avenue, is at a higher elevation, cross gradient, and 164 ft southeast of the subject sites. This property is identified in the EDR Database as a historic auto, reported spill incident, and has been assigned E-Designation, E-118, for underground gasoline storage tanks testing protocol by the New York City Department of City Planning as part of the Morrisania rezoning action (CEQR # 03DCP046X). Further review indicates three groundwater wells were installed at the adjacent property and gasoline contamination was reported in a single well adjacent to the gas station located at 1029 Washington Avenue.

- A review of the Municipal Records, indicated that Lot 51, located at 439 East 165th Street, has been assigned E-Designation, E-118, for underground gasoline storage tanks testing protocol as part of the Morrisania rezoning action (CEQR #03DCP046X).

## 1.5 	Summary of Work Performed under the Remedial Investigation

Rock performed the following scope of work:

- Conducted a Site inspection to identify AOCs and physical obstructions (i.e., structures, buildings, etc.).

- Installed nine (9) soil borings across the entire project Site, and collected twenty (20) soil samples for chemical analysis from the soil borings to evaluate soil quality.

- Installed five (5) groundwater monitoring wells throughout the Site to establish groundwater flow and collected six (6) groundwater samples for chemical analysis to evaluate groundwater quality; and,

- Installed eight (8) soil vapor probes around Site perimeter and collected eight (8) samples for chemical analysis.

# 1.6	Summary of Findings of Remedial Investigation

A remedial investigation was performed, and the results are documented in a companion document called “Remedial Investigation Report, 1049 Washington Avenue, Bronx, NY ”,  dated June 2023  (RIR).

Elevation of the property ranges from 26.00 to 28.42 feet North American Vertical Datum of 1988 (NAVD 88).

- Depth to groundwater ranges from 17.2 to 18.8 feet at the Site.

- Groundwater flow is generally from west to east beneath the Site.

- Depth to bedrock is unknown. Bedrock was not encountered during this RI. Based on regional information available to Rock, and a Geotechnical Investigation performed at the Site in February 2022, bedrock in the surrounding areas is anticipated at depths greater than 40 feet below grade.

- The stratigraphy of the site, from the surface down, consists of 8 feet of fill material, comprising of debris (trash, brick, broken rock and concrete) with fine to medium sand, underlain by red brown to tan medium to find grained sand with coarse sand, granules, and crushed rock throughout to a maximum depth of 17 ft bgs.

- Soil/fill samples collected during the RI were compared to 6 NYCRR NYSDEC Part 375.6-8(a) Unrestricted Use Soil Clean Up Objectives (SCOs), Residential, Restricted Residential, and Commercial Use SCOs.

- No VOCs, PCBs, or emerging contaminants were detected at concentrations above their respective Unrestricted Use SCOs.

- Five (5) SVOCs including, benzo(a)anthracene (max concentration of 11 mg/Kg), benzo(a)pyrene (max concentration of 11.7 mg/Kg), benzo(b)fluoranthene (max concentration of 9.52 mg/Kg), dibenzo (a, h)anthracene (max concentration of 2.02 mg/Kg), and indeno (1,2,3-cd)pyrene (max concentration of 7.59 mg/Kg), were detected at concentrations exceeding their Commercial Use SCO. Two (2) SVOCs including benzo(k)fluoranthene (max concentration of 8.65 mg/Kg) and chrysene (max concentration of 10.8 mg/Kg), were detected at concentrations exceeding their Restricted Residential Use SCOs. All maximum exceedances were located within SB-7 (0-2).  SVOCs in all deeper soil borings (10-12’) were detected at trace concentrations, below Unrestricted Use SCOs.

- Three (3) pesticides including 4,4’-DDD (max concentration of 0.00361 mg/Kg in DUP 2), 4,4’-DDE (max concentration of 0.00669 mg/Kg in SB-2 0-2), and 4,4’ DDT (max concentration of 0.0458 mg/Kg in DUP 2), were detected at concentrations exceeding their Unrestricted Use SCOs.

- Four (4) metals, including barium (max concentration 906 mg/Kg in SB-2 0-2), copper (max concentration of 416 mg/Kg in SB-7 0-2), lead (max concentration of 1,160 mg/Kg in Dup 2), and Nickel (max concentration of 1,230 m/Kg in SB-5 10-12), were detected at concentrations exceeding their Commercial Use SCOs. One (1) metal, mercury (max concentration of 1.28 mg/Kg in SB-6 0-2) was detected at a concentration above its Restricted Residential Use SCO. One (1) metal, cadmium (max concentration of 3.75 mg/Kg in SB-6 0-2) was detected at a concentration above its Residential Use SCO. Three (3) metals including, arsenic (max concentration of 15.5 mg/Kg in SB-6 0-2), selenium (max concentration of 6.22 mg/Kg in SB-1 0-2), and zinc (max concentration of 875 mg/Kg in SB-6 0-2), were detected at concentrations exceeding their Unrestricted Use SCOs.

- Groundwater samples collected during the RI were compared to the NYSDEC 6NYCRR Part 703.5 Class GA groundwater quality standards (GQS).

- No PCBs or 1,4 dioxane were detected in any of the groundwater samples at concentrations above their GQS.

- Two (2) VOCs, including PCE (max. of 10.8 ug/L in GW 1), and trichloroethylene (TCE) (max of 6.33 ug/L in GW-DUP), were detected at concentrations above their GQS.

- Six (6) SVOCs, including benzo(a)anthracene (max. concentration of 0.350 ug/L), benzo(a)pyrene (max. concentration of 0.380 ug/L in GW-2), benzo(b)fluoranthene (max concentration of 0.340 ug/L), benzo(k)fluoranthene (max concentration of 0.330 ug/L), bis(2-ethylhexl)phthalate (max concentration of 13.7 ug/L), chrysene (max concentration of 0.41 ug/L), indeno(1,2,3-cd)pyrene (max concentration of 0.23 ug/L), and naphthalene (max concentration of 10.1 ug/L) were detected at concentrations above their GQS. All maximum exceedances were detected within GW-2.

- One (1) pesticide, dieldrin (max concentration of 0.00449 g/L in GW-2), was detected at a concentration above the GQS.

- Four (4) metals including lead (max concentration of 62 ug/L in MW-2), magnesium (max concentration 69,100 ug/L in GW-4), manganese (max concentration 943 ug/L in GW-5), selenium (max concentration 74 ug/L in GW-3), and sodium (max concentration of 80,700 ug/L in GW-2), were detected at concentrations above their GQS.

- Two (2) dissolved metals, selenium (max concentration of 74.6 ug/L) and sodium (max concentration of 80,300 ug/L), were detected at concentrations above their GQS. All maximum exceedances were detected within GW-3.

- Two (2) emerging contaminates, including Perfluorooctanesulfonic acid (PFOS) (max concentration of 91.3 ug/L in GW-1) and Perfluorooctanoic acid (PFOA) (max concentration of 25.1 ug/L in GW-3), were detected at concentrations above their GQS.

- Soil vapor samples collected during the RI were compared to the compounds listed in table 3.1 of the Air Guideline Values derived by the NYSDOH located in New York State Department of Health (NYSDOH) Final Guidance for Evaluating Soil Vapor Intrusion, dated October 2006, and the revised NYSDOH Division Matrices, dated May 2017.

- Soil vapor results indicate elevated levels of petroleum related VOCs and chlorinated VOCs (CVOCs).

- Total concentrations of petroleum-related VOCs, benzene, toluene, ethylbenzene, and xylenes (BTEX) ranged from 254.1 micrograms per cubic meter (ug/m3) to 885 ug/m3, with the maximum concentration located in SV-6.

- CVOC concentrations include 1,1,1-Trichloroethane max concentration of 76 ug/m3), (cis-1,2-Dichloroethylene (max concentration of 12 ug/m3), PCE (max concentration of 1,300 ug/m3) and TCE (max concentration of 280 ug/m3) with maximum concentrations detected in SV-1. Methylene chloride (max concentration of 42 ug/m3 in SV-3) was also detected in the soil vapor samples. Maximum concentrations of TCA, TCE and PCE were detected in single soil vapor; location SV-1.

- 1,1-Dichloroethylene (max concentration of 0.67 ug/m3) and carbon tetrachloride (max concentration of 1.3 ug/m3) detected in SV-8 only.

- Vinyl chloride was not detected in any of the soil vapor samples collected from the site.

For more detailed results, consult the RIR. Based on an evaluation of the data and information from the RIR and this RAWP, disposal of significant amounts of hazardous waste is not suspected at this site.

# 2.0	Remedial Action Objectives

Based on the results of the RI, the following Remedial Action Objectives (RAOs) have been identified for this Site:

Soil

- Prevent direct contact with contaminated soil.

- Prevent migration of contaminants that would result in groundwater

Groundwater

- Prevent direct exposure to contaminated groundwater.

Soil Vapor

- Prevent exposure to contaminants in soil vapor.

- Prevent migration of soil vapor into dwelling and other occupied structures.

# 3.0	Remedial Alternatives Analysis

The goal of the remedy selection process is to select a remedy that is protective of human health and the environment taking into consideration the current, intended and reasonably anticipated future use of the property. The remedy selection process begins by establishing RAOs for media in which chemical constituents were found in exceedance of applicable standards, criteria and guidance values (SCGs). Remedial alternatives are then developed and evaluated based on the following ten criteria:

- Protection of human health and the environment;

- Compliance with SCGs;

- Short-term effectiveness and impacts;

- Long-term effectiveness and permanence;

- Reduction of toxicity, mobility, or volume of contaminated material;

- Implementability;

- Cost effectiveness;

- Community acceptance;

- Land use; and

- Sustainability.

As required, a Track 1 Unrestricted Use scenario is evaluated for the remedial action.  The following is a detailed description of the alternatives analyzed to address impacted media at the Site:

Alternative 1:

Selection of NYSDEC 6NYCRR Part 375 Unrestricted Use (Track 1) Soil Cleanup Objectives (SCOs).

- Removal of all soil/fill exceeding Track 1 Unrestricted Use SCOs throughout the Site and confirmation that Track 1 Unrestricted Use SCOs have been achieved with post-excavation endpoint sampling. Based on the results of the Remedial Investigation, it is expected that this alternative would be achieved by excavating for a new building cellar to a depth of 10 feet below  grade surface (bgs) and the elevator pit areas to a depth of 15 feet bgs. If soil/fill containing analytes at concentrations above Unrestricted Use SCOs is still present at the base of the excavation after removal of all soil required for construction of the new building's cellar level is complete, additional excavation would be performed to ensure complete removal of soil/ fill that does not meet Track 1 Unrestricted Use SCOs.

- No Engineering or Institutional Controls are required for a Track 1 cleanup. “As part of development,  a vapor barrier, composite cover system, and a passive sub-slab depressurization system (SSDS),would be installed to prevent potential exposures from soil vapor in the future.”

Alternative 2:

Establishment of Track 4 Site-Specific SCOs

- Removal of all soil/fill exceeding Track 4 Site-specific SCOs and confirmation that Track 4 Site-specific SCOs have been achieved with post-excavation end point sampling. Based on the results of the Remedial Investigation, it is expected that Track 4 SCOs would be achieved by excavating for the cellar foundation.  As part of development, soil beneath most of the site will be removed to a depth of 10 feet. If soil/fill containing analytes at concentrations above Track 4 Site-specific SCOs is still present at the base of the excavation, additional excavation would be performed to meet Track 4 Site-Specific SCOs.

- Removal of all soil/fill exceeding Track 4 Site-specific SCOs and confirmation that Track 4 Site-specific SCOs have been achieved with post-excavation end point sampling. Based on the results of the Remedial Investigation, it is expected that SCOs would be achieved by excavating for the construction of a new building's cellar level to a depth of approximately 10 feet across the entire building footprint. If soil/fill containing analytes at concentrations above Track 4 Site-Specific SCOs is still present at the base of the excavation, additional excavation would be performed to meet Track 4 Site-Specific SCOs.

- Placement of a composite cover system over the entire Site to prevent exposure to remaining soil/fill;

- Installation of a 20-mil thick vapor barrier system beneath the building slab and along foundation side walls to prevent potential exposures from soil vapor;

- Installation of a passive Sub Slab Depressurization System (SSDS); below the building cellar area to mitigate vapors potentially accumulating below the building’s foundation and prevent potential exposures from soil vapor;

- Establishment of use restrictions including prohibitions on the use of groundwater from the Site; prohibitions of restricted Site use, such as farming or vegetable gardening, to prevent future exposure pathways; and prohibition of a higher level of land use without OER approval;

- Establishment of an approved Site Management Plan (SMP) to ensure long-term management of these Engineering and Institutional Controls including the performance of periodic inspections and certification that the controls are performing as they were intended. The SMP will note that the property owner and the property owner’s successors and assigns must comply with the approved SMP; and

- The property will continue to be registered with an E-Designation at the NYC Department of Buildings.

## 3.1	Threshold Criteria
Protection of Public Health and the Environment

This criterion is an evaluation of the remedy’s ability to protect public health and the environment, and an assessment of how risks posed through each existing or potential pathway of exposure are eliminated, reduced or controlled through removal, treatment, and implementation of Engineering Controls or Institutional Controls.  Protection of public health and the environment must be achieved for all approved remedial actions.

Alternative 1 would be protective of human health and the environment by removing all soil/fill exceeding Track 1 Unrestricted Use SCO’s and groundwater protection standards, thus eliminating potential for direct contact with contaminated soil/fill once construction is complete and eliminating the risk of contaminants leaching into groundwater.

Alternative 2 would achieve comparable protections of human health and the environment through excavation and removal of most of the historic fill at the Site and by ensuring that remaining soil/fill on-Site meets Track 4 Site-Specific SCOs, as well as by placement of Institutional and Engineering Controls, including a composite cover system, vapor barrier, and a passive SSDS system. The composite cover system would prevent direct contact with any remaining on-Site soil/fill, the vapor barrier  and passive SSDS system will help mitigate the migration of soil vapors into the living spaces of the building. Implementing Institutional Controls including a Site Management Plan and continuing the E-designation would ensure that the composite cover system, vapor barrier and the passive SSDS system remains intact and protective of public health. The establishment of Track 4 Site-Specific SCOs would minimize the risk of contamination leaching into groundwater.

For both Alternatives, potential exposure to contaminated soils or groundwater during construction would be minimized by implementing a Construction Health and Safety Plan, an approved Soil/Materials Management Plan, and Community Air Monitoring Plan (CAMP). Potential contact with contaminated groundwater would be prevented as its use is prohibited by city laws and regulations. Potential future migration of off-Site soil vapors into the new building would be prevented by installing a vapor barrier below the building’s cellar slab, elevator pits, and around the foundation walls to grade and a passive SSDS below the building’s cellar slab.

# 3.2	Balancing Criteria

# Compliance with Standards, Criteria, and Guidance (SCGs)
This evaluation criterion assesses the ability of the alternative to achieve applicable standards, criteria and guidance.

Alternative 1 would achieve compliance with the remedial goals, chemical-specific SCGs and RAOs for soil through removal of soil to achieve Track 1 Unrestricted Use SCOs and Protection of Groundwater SCOs. Compliance with SCGs for soil vapor would also be achieved by installing a vapor barrier system below the new building's cellar slab and outside of foundation sidewalls, and a passive SSDS system below the cellar slab, as part of development. In addition, the cellar  of the building will contain a parking garage with high volume air exchange that conforms to the NYC Building Code.

Alternative 2 would achieve compliance with the remedial goals, chemical-specific SCG’s and RAOs for soil through removal of soil to meet Track 4 Site-Specific SCO’s. Compliance with SCG’s for soil vapor would also be achieved by installing a passive SSDS system beneath the building’s cellar slab and a vapor barrier system below the new building's cellar slab and continuing the vapor barrier outside of subgrade foundation walls. A Site Management Plan would ensure that these controls remained protective for the long term. In addition, the cellar of the building will contain a parking garage with high volume air exchange that conforms to the NYC Building Code and will mediate any potential accumulation of soil vapors inside the building.

Health and safety measures contained in the CHASP and Community Air Monitoring Plan (CAMP) will be implemented during Site redevelopment under this RAWP. For both Alternatives, focused attention on means and methods employed during the remedial action would ensure that handling and management of contaminated material would be in compliance with applicable SCGs. These measures will protect on-site workers and the surrounding community from exposure to Site-related contaminants.

# Short-Term Effectiveness and Impacts
This evaluation criterion assess the effects of the alternative during the construction and implementation phase until remedial action objectives are met.  Under this criterion, alternatives are evaluated with respect to their short-term effects during the remedial action on public health and the environment during implementation of the remedial action, including protection of the community, protection of onsite workers, and environmental impacts.

Both Alternative 1 and 2 have similar short-term effectiveness during their implementation, as each requires excavation of historic fill material. Both alternatives would result in short-term dust generation impacts associated with excavation, handling, load out of materials, and truck traffic. Short-term impacts may be  higher for Alternative 1 since excavation of greater amounts of historical fill material would take place. However, focused attention to means and methods during a Track 1 removal action, including community air monitoring and appropriate truck routing, would minimize the overall impact of these activities.

An additional short-term adverse impact and risks to the community associated with both remedial alternatives is increased truck traffic. Truck traffic will be routed on the most direct course using major thoroughfares where possible and flag persons will be used to protect pedestrians at Site entrances and exits.

The potential adverse impact to the community, workers and the environment for both alternatives would be minimized through implementation of control plans including a Construction Health and Safety Plan, a Community Air Monitoring Plan (CAMP) and a Soil/Materials Management Plan (SMMP), during all on-Site soil disturbance activities and would minimize the release of contaminants into the environment. Both alternatives provide short-term effectiveness in protecting the surrounding community by decreasing the risk of contact with on-Site contaminants. Construction workers operating under appropriate management procedures and a Construction Health and Safety Plan (CHASP) would provide protection from on-Site contaminants by using personal protective equipment would be worn consistent with the documented risks within the respective work zones.

# Long-term effectiveness and permanence

This evaluation criterion addresses the results of a remedial action in terms of its permanence and quantity/nature of waste or residual contamination remaining at the Site after response objectives have been met, such as permanence of the remedial alternative, magnitude of remaining contamination, adequacy of controls including the adequacy and suitability of Engineering Controls/Institutional Controls (ECs/ICs) that may be used to manage contaminant residuals that remain at the Site and assessment of containment systems and ICs that are designed to eliminate exposures to contaminants, and long-term reliability of ECs.

Alternative 1 would achieve long-term effectiveness and permanence related to on-Site contamination by permanently removing all impacted soil/fill above Track 1 Unrestricted Use SCO’s. Removal of on-Site contaminant sources will also prevent future groundwater contamination.

Alternative 2 would provide long-term effectiveness by removing most on-Site contamination and attaining Track 4 Site-Specific SCOs; installing a composite cover system across the Site; a vapor barrier system and a passive SSDS below the entire building foundation; maintaining use restrictions; establishing an SMP to ensure long-term management of ICs and ECs; and maintaining registration as an E-designated property to memorialize these controls for the long term. The SMP would ensure long-term effectiveness of all ECs and ICs by requiring periodic inspection and certification that these controls and restrictions continue to be in place and are functioning as they were intended, assuring that protections designed into the remedy continue to provide the required level of protection.

# Reduction of toxicity, mobility, or volume of contaminated material

This evaluation criterion assesses the remedial alternative's use of remedial technologies that permanently and significantly reduce toxicity, mobility, or volume of contaminants as their principal element. The following is the hierarchy of source removal and control measures that are to be used to remediate a Site, ranked from most preferable to least preferable: removal and/or treatment, containment, elimination of exposure, and treatment of source at the point of exposure. It is preferred to use treatment or removal to eliminate contaminants at a Site, reduce the total mass of toxic contaminants, causean  irreversible reduction in contaminants mobility, or reduce of total volume of contaminated media.

Alternative 1 will permanently eliminate the toxicity, mobility, and volume of contaminants from on-Site soil by removing all soil in excess of Track 1 Unrestricted Use SCO’s.

Alternative 2 would remove most of the historic fill at the Site, and all remaining on-Site soil/fill beneath the new building will meet Track 4 Site-Specific SCO’s.

Alternative 1 and Alternative 2 would remove a greater total mass of contaminants from the Site. The removal of soil/fill  for the excavation of building cellar to approximately 10  feet bgs and 15 feet bgs for the elevator pits for new development in both scenarios would lessen the difference in contaminant mass removal between these two alternatives.

# Implementability

This evaluation criterion addresses the technical and administrative feasibility of implementing an alternative and the availability of various services and materials required during its implementation, including technical feasibility of construction and operation, reliability of the selected technology, ease of undertaking remedial action, monitoring considerations, administrative feasibility (e.g., obtaining permits for remedial activities), and availability of services and materials.

The techniques, materials and equipment to implement both Alternatives 1 and 2 are readily available and have been proven to be effective in remediating the contaminants present on the Site. They use standard equipment and technologies that are well established in the industry. The reliability of each remedy is also high. There are no special difficulties associated with any of the activities proposed.

# Cost effectiveness

This evaluation criterion addresses the cost of alternatives, including capital costs (such as construction costs, equipment costs, and disposal costs, engineering expenses) and site management costs (costs incurred after remedial construction is complete) necessary to ensure the continued effectiveness of a remedial action.

Since historic fill at the Site was found to extend to a depth of up to 8 feet below grade surface during the RI, and the new building requires excavation of the entire Site to a depth of 10 feet bgs, the costs associated with both Alternative 1 and Alternative 2 will likely be comparable.  Costs associated with Alternative 1 could potentially be higher than Alternative 2 if soil with analytes above Track 1 Unrestricted Use SCOs are encountered below the excavation depth required for development. Additional costs would include installation of additional shoring/underpinning, disposal of additional soil, and import of clean soil for backfill.

The remedial plan would couple the remedial action with the redevelopment of the Site, lowering total costs. The remedial plan will also consider the selection of the most appropriate disposal facilities to reduce transportation and disposal costs during cleanup and redevelopment of the Site.

# Community Acceptance

This evaluation criterion addresses community opinion and support for remedial action. Observations here will be supplemented by public comment received on the RAWP.

This RAWP will be subject to a public review under the NYC VCP and will provide the opportunity for detailed public input on the remedial alternatives and the selected remedy. This public comment period will be considered by OER prior to approval of this plan. The Citizen Participation Plan for the project is provided in Appendix 2. Observations here will be supplemented by public comment received on the RAWP. Under both alternatives, the overall goals of the remedial program, to protect public health and the environment and eliminate potential contaminant exposures, have been broadly supported by citizens in NYC communities.

# Land use

This evaluation criterion addresses the proposed use of the property. This evaluation has considered reasonably anticipated future uses of the Site and takes into account: current use and historical and/or recent development patterns; applicable zoning laws and maps; NYS Department of State’s Brownfield Opportunity Areas (BOA) pursuant to section 970-r of the general municipal law; applicable land use plans; proximity to real property currently used for residential use, and to commercial, industrial, agricultural, and/or recreational areas; environmental justice impacts, Federal or State land use designations; population growth patterns and projections; accessibility to existing infrastructure; proximity of the site to important cultural resources and natural resources, potential vulnerability of groundwater to contamination that might emanate from the site, proximity to flood plains, geography and geology; and current Institutional Controls applicable to the Site.

The current, intended, and reasonably anticipated future land use of the Site and its surroundings are compatible with the selected remedy of soil remediation. The proposed future use of the Site includes a new 11-story mixed-use commercial and residential building fronting Washington Avenue. The proposed building will consist of a full build-out where the cellar level will be utilized for automobile parking of approximately 64 parking spaces, bicycle storage, building services, laundry, and a trash compactor room; the first floor will consist of commercial space, security services, lobby, a mailbox room, and parking access from East 165th Street; the second and third floors will be utilized as a day care center, with additional automobile parking (approximately 52 parking spaces) on the second floor; and the remaining eight floors (fourth through 11th) will be dedicated to approximately 104 residential units. Following remediation, the Site will meet either Track 1 Unrestricted Use or Track 4 Site-Specific SCOs, both of which are protective of public health and the environment for its planned residential use. The proposed use is compliant with the property’s zoning and is consistent with recent development patterns.

The areas surrounding the Site is bounded by mixed use commercial and residential buildings to the north, east, and south. The development would remediate the lot and provide a modern residential building. The proposed development would clean up the property and make it safer, create new employment opportunities, and other economic benefits from land revitalization.

Temporary short-term project impacts are being mitigated through site management controls and truck traffic controls during remediation activities. Following remediation, the Site will meet either Track 1 Unrestricted Use SCOs or Track 4 Site-Specific SCOs, both of which are protective of public health and the environment for its planned use.

The Site is not in close proximity to important cultural resources, including federal or state historic or heritage sites or Native American religious sites, natural resources, waterways, wildlife refuges, wetlands, or critical habitats of endangered or threatened species. The Site is in an urban area and not in proximity to fish or wildlife and neither alternative would result in any potential exposure pathways of contaminant migration affecting fish or wildlife. The remedial action is also protective of groundwater natural resources. The Site does not lie in a Federal Emergency Management Agency (FEMA)-designated flood plain. Both alternatives are equally protective of natural resources and cultural resources. Improvements in the current environmental condition of the property achieved by both alternatives considered in this plan are consistent with the City’s goals for cleanup of contaminated land.

# Sustainability of the Remedial Action

This criterion evaluates the overall sustainability of the remedial action alternatives and the degree to which sustainable means are employed to implement the remedial action including those that take into consideration NYC’s sustainability goals defined in PlaNYC.  Sustainability goals may include maximizing the recycling and reuse of non-virgin materials; reducing the consumption of virgin and non-renewable resources; minimizing energy consumption and greenhouse gas emissions; improving energy efficiency; and promotion of the use of native vegetation and enhancing biodiversity during landscaping associated with Site development.

The remedial plan would take into consideration the   shortest trucking routes during off-Site disposal of historic fill and other soils, which would reduce greenhouse gas emissions and conserve energy used to fuel trucks. The New York City Clean Soil Bank program may be used utilized for the reuse of import sols. To the extent practicable, energy efficient building materials, appliances, and equipment will be utilized to complete the development. While Alternative 2 would potentially result in lower energy usage based on reducing the volume of material transported off-site, the remedial alternatives are comparable with respect to the opportunity to achieve sustainable remedial action. A complete list of green remedial activities considered as part of the NYC VCP is included in the Sustainability Statement, included as Attachment C.

Selection of the Preferred Remedy

The preferred remedy for the Site is Alternative 2, Site-Specific (Track 4) Cleanup. Data generated during the site investigation support the conclusion that Alternative 2 is achievable as the entire site will be excavated for a new building cellar to a depth of 10 feet below grade surface (bgs), the elevator pits will be excavated to a maximum depth of 15 feet bgs. The Alternative 2 remedy will remove all soil/fill exceeding Track 4 Site-Specific Use SCOs throughout the Site, which will be confirmed with post-excavation sampling. If soil/fill containing analytes at concentrations above Track 4 SCOs is still present at the base or walls of the excavation after removal of all soil required for construction of the new building's cellar and elevator pit, additional excavation would be performed to ensure complete removal of soil/ fill that does not meet Track 4 SCOs.

As Engineering Controls are required for Track 4 Cleanups, a concrete slab covering the entire Site, a vapor barrier membrane, and passive SSDS would be installed as part of the standard building development.

Use restrictions will be imposed on the Site (including prohibitions on any use higher than Restricted Residential, e.g., the use of groundwater from the Site; prohibitions of restricted Site use, such as farming or vegetable gardening, to prevent future exposure pathways; and prohibition of a higher level of land use without OER approval), and the Site would continue to be encumbered with an E-designation for hazardous material.

# 4.0	Remedial Action

# 4.1	Summary of Preferred Remedial Action

The preferred remedial action alternative is Alternative 2, the Track  4 remedial actions. The preferred remedial action achieves protection of public health and the environment for the intended use of the property. The preferred remedial action will achieve all of the remedial action objectives established for the project and address applicable SCGs. The preferred remedial action is effective in both the short-term and long-term and reduces mobility, toxicity and volume of contaminants. The preferred remedial action alternative is cost effective and implementable and uses standards methods that are well established in the industry.

The proposed remedial action will consist of: Copy and paste from exec. summary

- Preparation of a Community Protection Statement and performance of all required NYC VCP Citizen Participation activities according to an approved Citizen Participation Plan.

- Performance of a Community Air Monitoring Program for particulates and volatile organic carbon compounds.

- Establishment of Track 4 Site-Specific Soil Cleanup Objectives (SCOs). Track 4 Site-Specific SCOs selected for this project are SVOCs at 100 ppm, Lead at 800 ppm, Copper at 270 mm, Barium at 650 ppm, and Mercury at 1.5 ppm.

- Site mobilization involving Site security setup, equipment mobilization, utility mark outs and marking & staking excavation areas.

- Completion of a Waste Characterization Study prior to excavation activities. Waste characterization soil samples will be collected at a frequency dictated by disposal facility(s).

- Excavation and removal of soil/fill exceeding Track 4 Site-Specific SCO.

- The entire footprint of the building area will be excavated to a depth of approximately 10  feet bgs  for development purpose. Two smaller portions of property will be excavated to a depth of 15 feet bgs for elevator pits. Approximately 11,000 CYC (16,500 tons)  of soil/fill will be removed from the Site and properly disposed of at an appropriately licensed or permitted facility.

- Screening of excavated soil/fill during intrusive work for indications of contamination by visual means, odor, and monitoring with a PID.

- Management of excavated materials including temporarily stockpiling and segregating in accordance with defined material types and to prevent co-mingling of contaminated material and non-contaminated material.

Removal of all UST’s that are encountered during soil/fill removal actions.

- Registration of tanks and reporting of any petroleum spills associated with UST’s and appropriate closure of these petroleum spills in compliance with applicable local, State and Federal laws and regulations.

- Transportation and off-Site disposal of all soil/fill material at licensed or permitted facilities in accordance with applicable laws and regulations for handling, transport, and disposal, and this plan. Sampling and analysis of excavated media as required by disposal facilities. Appropriate segregation of excavated media on-Site.

- Collection and analysis of nine (9) site-wide endpoint samples (EP1-EP9) across the building footprint to determine the performance of the remedy with respect to attainment of  SCOs.

- Import of materials to be used for backfill and cover in compliance with this plan and in accordance with applicable laws and regulations.

- Installation of a passive Sub-Slab Depressurization System (SSDS). The SSDS will consist of six loops installed within a permeable layer of ¾ inch bluestone with no fines beneath the building’s cellar slab. All six loops will be installed within the cellar level covering the entire building footprint. The SSDS loops will provide the correct coverage in accordance with USEPA sub-slab depressurization design specifications which recommend a separate vent loop for every 4,000 SF of slab area. The SSDS loops will be outfitted with a riser that will extend to the roof of the building. The exhaust will be placed at a minimum distance of 15ft from all air intakes. The SSDS is an Engineering Control for the remedial action.

- Construction of an engineered composite cover consisting of;

Cellar Slab: A 6-inch-thick concrete slab underlain with Stego Wrap 20-mil vapor barrier system underlain by an 8-inch layer of ASTM #5  ¾” bluestone with SSDS pipe underlain by residual soil;

- Installation of a vapor barrier system beneath the entire cellar slab, below/around the elevator pit, and behind all cellar walls to grade to mitigate soil vapor migration into the building. The vapor barrier system will consist of a Stego Wrap 20-mil vapor barrier system, or OER-approved equivalent system. All welds, seams and penetrations will be properly sealed to prevent preferential pathways for vapor migration.

- Performance of all activities required for the remedial action, including acquisition of required permits and attainment of pretreatment requirements, in compliance with applicable laws and regulations.

- Implementation of storm-water pollution prevention measures in compliance with applicable laws and regulations.

- Submission of a Remedial Action Report (RAR) that describes the remedial activities, certifies that the remedial requirements have been achieved, defines the Site boundaries, and lists any changes from this RAWP, and describes all Engineering and Institutional Controls to be implemented at the Site;

- Submission of an approved Site Management Plan (SMP) in the RAR for long-term management of residual contamination, including plans for operation, maintenance, monitoring, inspection and certification of Engineering and Institutional Controls and reporting at a specified frequency.

- As part of development, installation of a passive sub-slab depressurization system (SSDS) consisting of “a network of horizontal pipe set in the middle of a gas permeable layer immediately beneath the building slab and vapor barrier system. The horizontal piping will consist of fabric wrapped, perforated schedule 40, 4-inch PVC pipe connected to a 6-inch cast-iron riser pipe that penetrates the slab and travels through the building to the roof. The gas permeable layer will consist of a 8-inch-thick layer of ASTM 5 ¾ inch bluestone. The pipe will be finished at the roof line with a 6-inch goose neck pipe to prevent rain infiltration. The passive SSDS is an Engineering Control for the remedial action. The remedial engineer will certify in the RAR that the passive SSDS was designed and properly installed to establish a vacuum in the gas permeable layer and a negative (decreasing outward) pressure gradient across the building slab to prevent vapor migration into the building.

- The property will continue to be registered with an E-Designation at the NYC Buildings Department. Establishment of Engineering Controls and Institutional Controls in this RAWP and a requirement that management of these controls will include prohibition of the following: (1) vegetable gardening and farming; (2) use of groundwater without treatment rendering it safe for the intended use; (3) disturbance of residual contaminated material unless it is conducted in accordance with the SMP; and (4) higher level of land usage without OER-approval.

# 4.2	Soil Cleanup Objectives and Soil/ Fill Management

The following Track 4 Site-Specific SCO’s will be utilized for this project:

Contaminant			Site-Specific SCO’s

Total SVOCs                          	100 ppm

VOCs				Protection of GW Standards

Lead				800 ppm

Mercury			1.5 ppm

Barium				650 ppm

Copper                                    270 ppm

Soil and materials management on-Site and off-Site, including excavation, handling and disposal, will be conducted in accordance with the Soil/Materials Management Plan in Appendix D. Discrete contaminant sources (such as hotspots) identified during the remedial action will be identified by GPS or surveyed. This information will be provided in the Remedial Action Report.

# Soil/Fill Excavation and Removal

The redevelopment will require excavating for the building cellar to a maximum depth of 10 feet bgs and 15 feet bgs for the elevator pits. The location of planned excavations is shown in Figure 5. The total quantity of soil/fill expected to be excavated and disposed off-Site is 16,500 tons. For each disposal facility to be used in the remedial action, a letter from the developer/QEP to the receiving facility requesting approval for disposal and a letter back to the developer/QEP providing approval for disposal will be submitted to OER prior to any transport and disposal of soil at a facility.

The proposed disposal locations for Site-derived impacted materials are listed below.  Additional disposal locations established at a later date will be reported promptly to the OER Project Manager.

| Disposal Facility | Waste Type | Estimated Quantity |
| --- | --- | --- |
| Clean Earth, 24 Middlesex Avenue, Carteret, NJ 07008, New Jersey | Non-Haz Fill Material | 16,500 tons |

# End-point Sampling

End-point samples will be analyzed for compounds and elements as described below utilizing the following methodology:

- Volatile organic compounds by EPA Method 8260;

- Semi-volatile organic compounds by EPA Method 8270;

- Target Analyte List metals; and

- Pesticides/PCBs by EPA Method 8081/8082.

New York State ELAP certified labs will be used for all end-point sample analyses. Labs performing end-point sample analyses will be reported in the RAR. The RAR will provide a tabular and map summary of all end-point sample results and will include all data including non-detects and applicable standards and/or guidance values.

# Confirmation End-point Sampling

Removal actions for development purposes under this plan will be performed in conjunction with confirmation end-point soil sampling. Nine (9) confirmation samples (EP1-EP9) will be collected from the base of cellar and the and  elevator pits of the new building areas. Since Track 1 Unrestricted Use cleanup is feasible, endpoint samples will be analyzed for VOCs, SVOCs, pesticides, PCBs, and metals according to the analytical methods described above.

# Hotspot End-point Sampling

For any hotspots identified during this remedial program, including any hotspots identified during the remedial action, hotspot removal actions will be performed to ensure that hotspots are fully removed, and end-point samples will be collected at the following frequency:

- For excavations less than 20 feet in total perimeter, at least one bottom sample and one sidewall sample biased in the direction of surface runoff.

- For excavations 20 to 300 feet in perimeter:

  - For surface removals, one sample from the top of each sidewall for every 30 linear feet of sidewall and one sample from the excavation bottom for every 900 square feet of bottom area.

  - For subsurface removals, one sample from each sidewall for every 30 linear feet of sidewall and one sample from the excavation bottom for every 900 square feet of bottom area.

- For sampling of volatile organics, bottom samples should be taken within 24 hours of excavation and should be taken from the zero to six-inch interval at the excavation floor. Samples taken after 24 hours should be taken at six to twelve inches.

- For contaminated soil removal, post remediation soil samples for laboratory analysis should be taken immediately after contaminated soil removal. If the excavation is enlarged horizontally, additional soil samples will be taken pursuant to bullets 1-3 above.

Post-remediation end-point sample locations and depth will be biased towards the areas and depths of highest contamination identified during previous sampling episodes unless field indicators such as field instrument measurements or visual contamination identified during the remedial action indicate that other locations and depths may be more heavily contaminated. In all cases, post-remediation samples should be biased toward locations and depths of the highest expected contamination.

If either LNAPL and/or DNAPL are detected, appropriate samples will be collected for characterization and “fingerprint analysis” and required regulatory reporting (i.e., spills hotline) will be performed.

# Quality Assurance/Quality Control

The fundamental QA objective with respect to accuracy, precision, and sensitivity of analysis for laboratory analytical data is to achieve the QC acceptance of the analytical protocol. The accuracy, precision, and completeness requirements will be addressed by the laboratory for all data generated.

One duplicate sample for every 20 samples collected will be submitted to the approved laboratory for analysis of the same parameters. One trip blank will be submitted to the laboratory with each shipment of soil samples.

Collected samples will be appropriately packaged, placed in coolers, and shipped via overnight courier or delivered directly to the analytical laboratory by field personnel. Samples will be containerized in appropriate laboratory-provided glassware and shipped in plastic coolers. Samples will be preserved using ice or “cold-paks” to maintain a temperature of 4 degrees C.

Dedicated disposable sampling materials will be used for the collection endpoint samples, eliminating the need to prepare field equipment (rinsate) blanks. However, if non-disposable equipment is used, (stainless steel scoop, etc.) field rinsate blanks will be prepared at the rate of 1 for every eight samples collected. Decontamination of non-dedicated sampling equipment will consist of the following:

    - Gently tap or scrape to remove adhered soil.

    - Rinse with tap water

    - Wash with Alconox® detergent solution and scrub

    - Rinse with tap water

    - Rinse with distilled or deionized water

Prepare field blanks by pouring distilled or deionized water over decontaminated equipment and collecting the water in laboratory-provided containers. Trip blanks will be used whenever samples are transported to the laboratory for analysis of VOCs. Trip blanks will not be used for samples to be analyzed for metals, SVOCs or pesticide prepared and submitted for analysis every 20 samples.

# Import of Soils

Soil import is not planned for this project. However, an import of ASTM 5 ¾ inch blue stone will be required for the gas permeable layer during installation of the passive sub-slab depressurization system.

The estimated quantity of ASTM5 ¾ inch bluestone  to be imported onto the Site is 900 tons. A map of bluestone placement locations is shown in Figure 12.

If soil import is planned, OER will be informed prior to import and import of soils onto the property will be performed in conformance with the Soil/Materials Management Plan in Attachment D.  Imported soil will meet the lower of:

- Track 2 Restricted Residential use SCO’s; and,

- Groundwater Protection Standards in part 375-6.8

# Reuse of Onsite Soils

## Reuse of soil is not anticipated at the Site. However, if needed, reuse of on-Site soil already on Site will be performed in conformance with the Soil/management Plan in Attachment D. Reuse soils will meet Track 1 Unrestricted Use SCOs. OER will be notified prior to any reuse of on-Site soil.

# 4.3	Engineering Controls

Engineering Controls will be employed in the remedial action to address residual contamination remaining at the site. The Site has three (3)  Primary Engineering Control Systems. These are:

- Composite Cover System

- Soil Vapor Barrier System

- Passive Sub-Slab Depressurization System

# Composite Cover System

Exposure to residual soil/fill will be prevented by an engineered, composite cover system to be built on the Site. This composite cover system will be comprised of the following:

Cellar slab: 6-inch-thick concrete slab underlain with Stego Wrap 20-mil vapor barrier system underlain by a 8-inch layer of ASTM5  ¾” bluestone with SSDS pipe underlain by residual soil.

Figure 8 shows the typical design for each remedial cover type used on this Site.

The composite cover system will be a permanent engineering control. The system will be inspected, and its performance certified at specified intervals as required by this RAWP and the Site Management Plan.  A Soil and Materials Management Plan will be included in the Site Management Plan and will outline the procedures to be followed in the event that the composite cover system and underlying residual soil/fill is disturbed after the remedial action is complete. Maintenance of this composite cover system will be described in the Site Management Plan in the Remedial Action Report (RAR).

# Vapor Barrier System

Migration of soil vapor from onsite or offsite sources into the building will be mitigated with a combination of building slab and vapor barrier. The vapor barrier will consist of a Stego Wrap 20-mil vapor barrier system or OER-approved equivalent, installed below the entire building’s cellar slab, below/around the elevator pits and behind all cellar walls to grade.  The vapor barrier system will be installed prior to pouring the building’s concrete slab.

The vapor barrier will extend throughout the area occupied by the footprint of the new building and up the foundation sidewalls and will be installed in accordance with manufacturer specifications.
The specifications for installation will be provided to the construction management company and the foundation contractor or installer of the liner. The specifications state that all vapor barrier seams, penetrations, and repairs will be sealed either by the tape method or weld method, according to the manufacturer's recommendations and instructions.

The project's Professional Engineer licensed by the State of  New York will have primary direct responsibility for overseeing the implementation of the vapor barrier. The extent of the proposed vapor barrier is provided in Figure 8 and Figure 9. Product specification sheets are provided in Attachment F.

The Remedial Action Report will include as-built drawings and diagrams; manufacturer documentation;  photographs and

a PE-certified letter (on company letterhead) from the primary contractor responsible for installation oversight and field inspections and a copy of the manufacturer’s certificate of warranty.

The Vapor Barrier System is a permanent engineering control and will be inspected and its performance certified at specified intervals as required by this RAWP and the Site Management Plan. A Soil and Materials Management Plan will be included in the Site Management Plan and will outline the procedures to be followed in the event that the composite cover system and underlying vapor barrier system are disturbed after the remedial action is complete. The maintenance of these systems will be described in the Site Management Plan in the Remedial Action Report.

# Passive Sub-Slab Depressurization System

Migration of soil vapor into the building will be mitigated with the construction of  a passive Sub-Slab Depressurization System (SSDS). The SSDS will consist of six loops installed within porous granular material; ASTM 5 ¾ inch bluestone with no fines; beneath the cellar foundation. The SSDS loops will provide the correct coverage in accordance with USEPA sub-slab depressurization design specifications which recommend a separate vent loop for every 4,000 SF of slab area. The SSDS loops will be outfitted with a riser that will extend to the roof of the building. The exhaust will be placed at a minimum distance of 10-15 feet from all air intakes.

The SSDS is a permanent engineering control. The system will be inspected, and its performance certified at specified intervals as required by this RAWP and the Site Management Plan. Maintenance of this SSDS will be described in the Site Management Plan in the Remedial Action Report. The location and layout of the SSDS is shown in Figure10. A typical section of the system is shown in Figure 11.

# 4.4	Institutional Controls

A series of Institutional Controls (IC’s) are required under this Remedial Action to assure permanent protection of public health by elimination of exposure to residual materials. These ICs define the program to operate, maintain, inspect and certify the performance of Engineering Controls and Institutional Controls on this property. Institutional Controls would be implemented in accordance with a Site Management Plan included in the final Remedial Action Report (RAR). Institutional Controls would be:

- Continued registration of the E-Designation at the NYC Department of Buildings.  This RAWP includes a description of all ECs and ICs and summarizes the requirements of the SMP which will note that the property owner and property owner’s successors and assigns must comply with the approved SMP.

- Submittal of a SMP in the RAR for approval by OER that provides procedures for appropriate operation, maintenance, inspection, and certification of ECs and IC’s. SMP will require that the property owner and property owner’s successors and assigns will submit to OER a periodic written statement that certifies that: (1) controls employed at the Site are unchanged from the previous certification or that any changes to the controls were approved by OER; and, (2) nothing has occurred that impairs the ability of the controls to protect public health and environment or that constitute a violation or failure to comply with the SMP.  OER retains the right to enter the Site in order to evaluate the continued maintenance of any controls.  This certification shall be submitted at a frequency to be determine by OER in the SMP and will comply with RCNY §43-1407(l)(3).

- Vegetable gardens and farming on the Site are prohibited in contact with residual soil materials;

- Use of groundwater underlying the Site is prohibited without treatment rendering it safe for its intended use;

- All future activities on the Site that will disturb residual material must be conducted pursuant to the soil management provisions in an approved SMP;

- The Site will be utilized  for residential and commercial use and will not be used for a higher level of use without prior approval by OER.

# 4.5	Site Management Plan

Site Management is the last phase of remediation and begins with the approval of the Remedial Action Report and issuance of the Notice of Completion (NOC) for the Remedial Action. The Site Management Plan (SMP) describes appropriate methods and procedures to ensure implementation of all ECs and ICs that are required by this RAWP. The Site Management Plan is submitted as part of the RAR but will be written in a manner that allows its use as an independent document. Site Management continues until terminated in writing by OER. The property owner is responsible to ensure that all Site Management responsibilities defined in the Site Management Plan are implemented.

The SMP will provide a detailed description of the procedures required to manage residual soil/fill left in place following completion of the remedial action in accordance with the Voluntary Cleanup Agreement with OER. This includes a plan for: (1) implementation of EC’s and ICs; (2) operation and maintenance of EC’s; (3) inspection and certification of IC’s and EC’s.

Site management activities and EC/IC certification will be scheduled by OER on a periodic basis to be established in the RAR and the SMP and will be subject to review and modification by OER. The Site Management Plan will be based on a calendar year and certification reports will be due for submission to OER by July 30 of the year following the reporting period.

# 4.6	Qualitative Human Health Exposure Assessment

The objective of the qualitative exposure assessment is to identify potential receptors and pathways for human exposure to the contaminants of concern (COC) that are present at, or migrating from, the Site. The identification of exposure pathways describes the route that the COC takes to travel from the source to the receptor. An identified pathway indicates that the potential for exposure exists; it does not imply that exposures actually occur.

Data and information reported in the Remedial Investigation Report (RIR) are sufficient to complete a Qualitative Human Health Exposure Assessment (QHHEA) for this project. As part of the VCP process, a QHHEA was performed to determine whether the Site poses an existing or future health hazard to the Site’s exposed or potentially exposed population. The sampling data from the RI were evaluated to determine whether there is any health risk under current and future conditions by characterizing the exposure setting, identifying exposure pathways, and evaluating contaminant fate and transport. This QHHEA was prepared in accordance with Appendix 3B and Section 3.3 (b) 8 of the NYSDEC Draft DER-10 Technical Guidance for Site Investigation and Remediation.

# Known and Potential Contaminant Sources:

Based on the results of the RIR, the contaminants of concern are:

Soil:

- SVOCs including benzo(a)anthracene, benzo(a)pyrene, benzo(b), dibenzo(a, h)anthracene, and indeno (1,2,3-cd)pyrene, were detected at concentrations exceeding their Commercial Use SCO and benzo(k)fluoranthene and chrysene were detected at concentrations exceeding their Restricted Residential Use SCOs. All maximum exceedances were located within SB-7 (0-2).

- Pesticides including 4-4’-DDD, 4,4’-DDE, and 4,4’ DDT, were detected at concentrations exceeding their Unrestricted Use SCOs.

- Metals including barium, copper, and Nickel, were detected at concentrations exceeding their Commercial Use SCO and mercury was detected at a concentration above its Restricted Residential Use SCO. Cadmium was detected at a concentration above its Residential Use SCO and arsenic, selenium and zinc, were detected at concentrations exceeding their Unrestricted Use SCOs.

Groundwater:

- VOCs including PCE and trichloroethylene (TCE) were detected at concentrations above their GQS.

- SVOCs including benzo(a)anthracene, benzo(a)pyrene, benzo(b)fluoranthene, benzo(k)fluoranthene, bis(2-ethylhexl) phthalate, chrysene, indeno(1,2,3-cd) pyrene, and naphthalene were detected at concentrations above their GQS. All maximum exceedances were detected within GW-2.

- Pesticides including dieldrin were detected at a concentration above the GQS.

- Metals including lead, magnesium, manganese, selenium, and sodium, were detected at concentrations above their GQS.

- Dissolved metals including selenium and sodium, were detected at concentrations above their GQS. All maximum exceedances were detected within GW-3.

- Emerging contaminates, including Perfluorooctanesulfonic acid (PFOS) and Perfluorooctanoic acid (PFOA), were detected at concentrations above their GQS.

Soil Vapor:

- Soil vapor results indicate elevated levels of petroleum related VOCs and chlorinated VOCs (CVOCs).

- Total concentrations of petroleum-related VOCs, benzene, toluene, ethylbenzene, and xylenes (BTEX) ranged from 254.1 micrograms per cubic meter (ug/m3) to 885 ug/m3, with the maximum concentration located in SV-6.

- CVOC concentrations include 1,1,1-Trichloroethane (max of 76 ug/m3), (cis-1,2-Dichloroethylene (max of 12 ug/m), PCE (max of 1,300 ug/m3) and TCE (max of 280 ug/m3) with maximum concentrations detected in SV-1. Methylene chloride (max of 42 ug/m3) was also detected in the soil vapor samples. Maximum concentrations of TCA, TCE and PCE were detected in single soil vapor; location SV-1.

- 1,1-Dichloroethylene (max of 0.67 ug/m) and carbon tetrachloride (max of 1.3 ug/m3) were detected in SV-8 only.

# Nature, Extent, Fate, and Transport of Contaminants

Soil: Historic fill material with elevated levels of SVOCs, metals and pesticides was encountered from the surface to a depth of approximately 8 feet below surface grade.  Most the soil exceedances (exceeding UUSCOs and RRSCOs) were from samples collected from the shallow interval; however, a few metals and two pesticides were detected at levels exceeding SCOs in deeper interval soil.  Metals exceeding Track 1-UUSCOs in deep interval soil samples (10-12 ft and 15 to 17 ft) included Arsenic (max. 15,5 mg/kg), Copper (max. of 80 mg/kg), Lead (max. of 292 mg/kg), Selenium (max. of 4.8 mg/kg) and Zinc (max. of 291 mg/kg).  No metals exceeded Track 2 – RRSCOs in deep interval samples.   The pesticides, 4,4’-DDT (max. 0.0305 ug/kg) and 4,4’-DDE (max. of 0.00531 ug/kg) were detected above UUSCOs in deep interval soil.  No VOCs or PCBs were detected above SCOs in either shallow or deep interval samples.

Groundwater: VOCs including PCE and trichloroethylene (TCE) were detected at concentrations above their GQS. The source of these contaminants could potentially be from the historic presence of fuel storage tanks and an auto repair, the historic use of the Site by Ambassador Fuel, Electric & Die Corp., or the adjacent auto repair with known spill history.  Impacts could also be associated with properties in the immediate vicinity with environmental conditions.

CVOCs in groundwater present a concern with respect to vapor intrusion.  The migratory pathway is from vapors volatilizing out of groundwater and permeating through any onsite structures, thereby impacting indoor air quality.

Soil Vapor:  Soil vapor results during the RI indicated elevated levels of petroleum related VOCs and chlorinated VOCs (CVOCs). The source of the soil vapors could be associated with the historic presence of fuel storage tanks and an auto repair onsite, the historic use of the Site by Ambassador Fuel, Electric & Die Corp., or the adjacent auto repair with known spill history.  Impacts could also be associated with properties in the immediate vicinity with environmental conditions.

The migratory pathway for soil gas is to permeate through any onsite structures, thereby impacting indoor air quality.

# Receptor Populations

On-Site Receptors:  The site is currently vacant and contains a one-story commercial warehouse with a partial cellar and associated parking lots.  Onsite receptors are limited to  trespassers, site representatives and visitors granted access to the property. During construction, potential on-site receptors include construction workers, site representatives, and visitors. Under proposed future conditions, potential on-site receptors include adult and child building residents, workers and visitors.

Off-Site Receptors:  Potential off-site receptors within a 500-foot radius of the Site include adult and child residents; commercial and construction workers; pedestrians; and trespassers based on the following land uses within 500 feet of the Site:

- Commercial Businesses – existing and future

- Residential Buildings – existing and future

- Building Construction/ Renovation – existing and future

- Pedestrians, Trespassers, Cyclists – existing and future

- Schools – existing and future

# Potential Routes of Exposure

Three potential primary routes exist by which chemicals can enter the body:  ingestion, inhalation, and dermal absorption.  Exposure can occur based on the following potential media:

- Ingestion of groundwater or fill/ soil;

- Inhalation of vapors or particulates; and

- Dermal absorption of groundwater or fill/ soil.

# Potential Exposure Points

Current Conditions:  The site is currently capped with asphalt and there are no potential exposure pathways from ingestion, inhalation, or dermal absorption of soil/ fill. Groundwater is not exposed at the site. The site is served by the public water supply and groundwater is not used at the site for potable supply and there is no potential for exposure.  The Site is currently vacant and contains a one-story warehouse with a partial  cellar and  associated parking slots.

Construction/ Remediation Conditions:  Once redevelopment activities begin, onsite workers will come into direct contact with surface and subsurface soils as a result of on-Site construction and excavation activities. On-Site construction workers potentially could ingest, inhale or have dermal contact with exposed impacted soil and fill. Similarly, off-Site receptors could be exposed to dust and vapors from on-Site activities. Due to the depth of groundwater, direct contact with groundwater is not expected. During construction, on-Site and off-Site exposures to contaminated dust from on-Site will be addressed through the Soil/Materials Management Plan, dust controls, and through the implementation of the Community Air-Monitoring Program and a Construction Health and Safety Plan.

Proposed Future Conditions: Under future remediated conditions, all soils in excess of Track 4 SCOs will be removed. The site will be fully capped, preventing potential direct exposure to soil and groundwater remaining in place, and engineering controls (vapor barrier, composite cover system, and a passive SSDS) will prevent any potential exposure due to inhalation by preventing soil vapor intrusion.  The site is served by the public water supply, and groundwater is not used at the site. There are no plausible off-site pathways for oral, inhalation, or dermal exposure to contaminants derived from the Site.

# Overall Human Health Exposure Assessment

There are potential complete exposure pathways for the current site condition that require mitigation during the implementation of the remedy.  Under current conditions, on-Site exposure pathways exist for those with access to the Site and trespassers. During remedial construction, on-Site and off-Site exposures to contaminated dust from historic fill material will be addressed through dust controls, and through the implementation of the Community Air Monitoring Program, the Soil/Materials Management Plan, and a Construction Health and Safety Plan. There is no complete exposure pathway under future conditions after the Site is developed as all soil above Track 4 Site Specific SCOs will have been removed and a vapor barrier system, passive SSDS, and composite cover system will be installed. The SSDS and vapor barrier system will prevent potential vapor intrusion. The composite cover system and use restrictions will prevent contact with residual soil or groundwater and continued protection after the remedial action will be achieved by the implementation of Site management including periodic inspection and certification of the performance of remedial controls. Potential post-construction use of groundwater is not considered an option because groundwater in this area of New York City is not used as a potable water source. There are no surface waters in close proximity to the Site that could be impacted or threatened. This assessment takes into consideration the reasonably anticipated use of the Site, which includes a residential structure, Site-wide impervious surface cover cap, and a vapor barrier for the building.

# 5.0	Remedial Action Management

# 5.1	Project Organization and Oversight

Principal personnel who will participate in the remedial action include Robert M. Bennett, Senior Project Manager, Roselina Quadros, Assistant Project Manager and Jason Gellati, Field Operations Officer-Rock Enviro. The Professional Engineer (PE) and Qualified Environmental Professionals (QEP) for this project are___ and Robert M. Bennett-Rock Enviro.

# 5.2	Site Security

Site access will be controlled by a typical, green-painted wooden construction fence, which will run along the entire property boundary surrounding the entire Site.

# 5.3	Work Hours

The hours for operation of cleanup will comply with the NYC Department of Buildings construction code requirements or according to specific variances issued by that agency.  The hours of operation will be conveyed to OER during the pre-construction meeting.

# 5.4	Construction Health and Safety Plan

The Health and Safety Plan is included in Appendix 5. The Site Safety Coordinator will be Jason Gellati, Field Manager for Rock Enviro. Remedial work performed under this RAWP will be in full compliance with applicable health and safety laws and regulations, including Site and OSHA worker safety requirements and HAZWOPER requirements. Confined space entry, if any, will comply with OSHA requirements and industry standards and will address potential risks. The parties performing the remedial construction work will ensure that performance of work is in compliance with the HASP and applicable laws and regulations. The HASP pertains to remedial and invasive work performed at the Site until the issuance of the Notice of Completion.

All field personnel involved in remedial activities will participate in training required under 29 CFR 1910.120, such as 40-hour hazardous waste operator training and annual 8-hour refresher training.  The Site Safety Officer will be responsible for maintaining workers’ training records.

Personnel entering any exclusion zone will be trained in the provisions of the HASP and will comply with all requirements of 29 CFR 1910.120.  Site-specific training will be provided to field personnel. Additional safety training may be added depending on the tasks performed.  Emergency telephone numbers will be posted at the site location before any remedial work begins.  A safety meeting will be conducted before each shift begins. Topics to be discussed include task hazards and protective measures (physical, chemical, environmental); emergency procedures; PPE levels and other relevant safety topics. Meetings will be documented in a logbook or specific form.

An emergency contact sheet with names and phone numbers is included in the CHASP.  That document will define the specific project contacts for use in case of emergency.

# 5.5	Community Air Monitoring Plan

Real-time air monitoring for volatile organic compounds (VOCs) and particulate levels at the perimeter of the exclusion zone or work area will be performed. Continuous monitoring will be performed for all ground intrusive activities and during the handling of contaminated or potentially contaminated media. Ground intrusive activities include, but are not limited to, soil/waste excavation and handling, test pit excavation or trenching, and the installation of soil borings or monitoring wells.

Periodic monitoring for VOCs will be performed during non-intrusive activities such as the collection of soil and sediment samples or the collection of groundwater samples from existing monitoring wells. Periodic monitoring during sample collection, for instance, will consist of taking a reading upon arrival at a sample location, monitoring while opening a well cap or overturning soil, monitoring during well bailing/purging, and taking a reading prior to leaving a sample location. Depending upon the proximity of potentially exposed individuals, continuous monitoring may be performed during sampling activities.  Examples of such situations include groundwater sampling at wells on the curb of a busy urban street, in the midst of a public park, or adjacent to a school or residence. Exceedances of action levels observed during performance of the Community Air Monitoring Plan (CAMP) will be reported to the OER Project Manager and included in the Daily Report.

# VOC Monitoring, Response Levels, and Actions

Volatile organic compounds (VOCs) will be monitored at the downwind perimeter of the immediate work area (i.e., the exclusion zone) on a continuous basis during invasive work. Upwind concentrations will be measured at the start of each workday and periodically thereafter to establish background conditions. The monitoring work will be performed using equipment appropriate to measure the types of contaminants known or suspected to be present. The equipment will be calibrated at least daily for the contaminant(s) of concern or for an appropriate surrogate. The equipment will be capable of calculating 15-minute running average concentrations, which will be compared to the levels specified below.

- If the ambient air concentration of total organic vapors at the downwind perimeter of the work area or exclusion zone exceeds 5 parts per million (ppm) above background for the 15-minute average, work activities will be temporarily halted and monitoring continued.  If the total organic vapor level readily decreases (per instantaneous readings) below 5 ppm over background, work activities will resume with continued monitoring.

- If total organic vapor levels at the downwind perimeter of the work area or exclusion zone persist at levels in excess of 5 ppm over background but less than 25 ppm, work activities will be halted, the source of vapors identified, corrective actions taken to abate emissions, and monitoring continued. After these steps, work activities will resume provided that the total organic vapor level 200 feet downwind of the exclusion zone or half the distance to the nearest potential receptor or residential/commercial structure, whichever is less - but in no case less than 20 feet, is below 5 ppm over background for the 15-minute average.

- If the organic vapor level is above 25 ppm at the perimeter of the work area, activities will be shutdown.

All 15-minute readings must be recorded and be available for OER personnel to review. Instantaneous readings, if any, used for decision purposes will also be recorded.

# Particulate Monitoring, Response Levels, and Actions

Particulate concentrations will be monitored continuously at the upwind and downwind perimeters of the exclusion zone at temporary particulate monitoring stations. The particulate monitoring will be performed using real-time monitoring equipment capable of measuring particulate matter less than 10 micrometers in size (PM-10) and capable of integrating over a period of 15 minutes (or less) for comparison to the airborne particulate action level. The equipment will be equipped with an audible alarm to indicate exceedance of the action level. In addition, fugitive dust migration should be visually assessed during all work activities.

- If the downwind PM-10 particulate level is 100 micrograms per cubic meter (mcg/m3) greater than background (upwind perimeter) for the 15-minute period or if airborne dust is observed leaving the work area, then dust suppression techniques will be employed.  Work will continue with dust suppression techniques provided that downwind PM-10 particulate levels do not exceed 150 mcg/m3 above the upwind level and provided that no visible dust is migrating from the work area.

- If, after implementation of dust suppression techniques, downwind PM-10 particulate levels are greater than 150 mcg/m3 above the upwind level, work will be stopped, and a re-evaluation of activities initiated. Work will resume provided that dust suppression measures and other controls are successful in reducing the downwind PM-10 particulate concentration to within 150 mcg/m3 of the upwind level and in preventing visible dust migration.

All readings will be recorded and be available for OER personnel to review.

## 5.6	Agency Approvals

All permits or government approvals required for remedial construction have been or will be obtained prior to the start of remedial construction. Approval of this RAWP by OER does not constitute satisfaction of these requirements and will not be a substitute for any required permit.

## 5.7	Site Preparation

# Pre-Construction Meeting

OER will be invited to attend the pre-construction meeting at the Site with all parties involved in the remedial process prior to the start of remedial construction activities.

# Mobilization

Mobilization will be conducted as necessary for each phase of work at the Site.  Mobilization includes field personnel orientation, equipment mobilization (including securing all sampling equipment needed for the field investigation), marking/staking sampling locations and utility mark-outs.  Each field team member will attend an orientation meeting to become familiar with the general operation of the Site, health and safety requirements, and field procedures.

# Utility Marker Layouts, Easement Layouts

The presence of utilities and easements on the Site will be fully investigated prior to the performance of invasive work such as excavation or drilling under this plan by using, at a minimum, the One-Call System (811). Underground utilities may pose electrocution, explosion, or other hazard during excavation or drilling activities.  All invasive activities will be performed in compliance with applicable laws and regulations including NYC Building Code to assure safety. Utility companies and other responsible authorities will be contacted to locate and mark the locations, and a copy of the Mark-Out Ticket will be retained by the contractor prior to the start of drilling, excavation or other invasive subsurface operations.  Overhead utilities may also be present within the anticipated work zones.  Electrical hazards associated with drilling in the vicinity of overhead utilities will be prevented by maintaining a safe distance between overhead power lines and drill rig masts.

Proper safety and protective measures pertaining to utilities and easements, and compliance with all laws and regulations will be employed during invasive and other work contemplated under this RAWP. The integrity and safety of on-Site and off-Site structures will be maintained during all invasive, excavation or other remedial activity performed under the RAWP.

# Dewatering

Dewatering is not anticipated during remediation and construction at this Site as groundwater is present at a depth of 19  feet bgs and the maximum proposed excavation depth is at 10 feet bgs for the cellar and 15 feet bgs for the elevator pits. If dewatering is required, the water will be disposed into the New York City combined sanitary/storm sewer system. A permit to discharge will be obtained from the New York City Department of Environmental Protection (NYCDEP). As part of the permit to discharge, the location of discharge will be based on the Site-Specific requirements of the DEP. The need for pretreatment will be determined by DEP’s requirements for the discharge permit. If pretreatment is required by the DEP, it will be performed in accordance with the requirements of the DEP.

# Equipment and Material Staging

Equipment and materials will be stored and staged in a manner that complies with applicable laws and regulations. Staging locations will be reported to OER prior to the start of the remedial action.

# Stabilized Construction Entrance

Steps will be taken to ensure that trucks departing from the site will not track soil, fill or debris off-Site. Such actions may include use of cleaned asphalt or concrete pads or use of stone or other aggregate-based egress paths between the truck inspection station and the property exit. Measures will be taken to ensure that adjacent roadways will be kept clean of project related soil, fill and debris.

# Truck Inspection Station

An outbound-truck inspection station will be set up close to the Site exit.  Before exiting the Site, trucks will be required to stop at the truck inspection station and will be examined for evidence of contaminated soil on the undercarriage, body, and wheels.  Soil and debris will be removed.  Brooms, shovels and clean water will be utilized for the removal of soil from vehicles and equipment, as necessary.

# Extreme Storm Preparedness and Response Contingency Plan

Damage from flooding or storm surge can include dislocation of soil and stockpiled materials, dislocation of site structures and construction materials and equipment, and dislocation of support of excavation (SOE) structures. Damage from wind during an extreme storm event can create unsafe or unstable structures, damage safety structures and cause downed power lines creating dangerous site conditions and loss of power. In the event of emergency conditions caused by an extreme storm event, the enrollee will undertake the following steps for site preparedness prior to the event and response after the event.

# Storm Preparedness

Preparations in advance of an extreme storm event will include the following: containerized hazardous materials and fuels will be removed from the property; loose materials will be secured to prevent dislocation and blowing by wind or water; heavy equipment such as excavators and generators will be removed from excavated areas, trenches and depressions on the property to high ground or removed from the property; an inventory of the property with photographs will be performed to establish conditions for the site and equipment prior to the event; stockpile covers for soil and fill will be secured by adding weights such as sandbags for added security and worn or ripped stockpile covers will be replaced with competent covers; stockpiled hazardous wastes will be removed from the property;  stormwater management systems will be inspected and fortified, including, as necessary: clean and reposition silt fences, hay bales; clean storm sewer filters and traps; and secure and protect pumps and hosing.

# Storm Response

At the conclusion of an extreme storm event, as soon as it is safe to access the property, a complete inspection of the property will be performed. A site inspection report will be submitted to OER at the completion of site inspection and after the site security is assessed. Site conditions will be compared to the inventory of site conditions and material performed prior to the storm event and significant differences will be noted. Damage from storm conditions that result in acute public safety threats, such as downed power lines or imminent collapse of buildings, structures or equipment will be reported to public safety authorities via appropriate means such as calling 911. Petroleum spills will be reported to NYS DEC within 2 hours of identification and consistent with State regulations. Emergency and spill conditions will also be reported to OER. Public safety structures, such as construction security fences will be repaired promptly to eliminate public safety threats. Debris will be collected and removed. Dewatering will be performed in compliance with existing laws and regulations and consistent with emergency notifications, if any, from proper authorities. Eroded areas of soil including unsafe slopes will be stabilized and fortified. Dislocated materials will be collected and appropriately managed. Support of excavation structure will be inspected and fortified as necessary. Impacted stockpiles will be contained and damaged stockpile covers will be replaced. Stormwater control systems and structures will be inspected and maintained as necessary. If soil or fill materials are discharged off site to adjacent properties, property owners and OER will be notified, and corrective measure plan designed to remove and clean dislocated material will be submitted to OER and implemented following approval by OER and granting of site access by the property owner. Impacted offsite areas may require characterization based on site conditions, at the discretion of OER. If onsite petroleum spills are identified, a qualified environmental professional will determine the nature and extent of the spill and report to NYS DEC’s spill hotline at DEC 800-457-7362 within statutory defined timelines. If the source of the spill is ongoing and can be identified, it should be stopped if this can be done safely. Potential hazards will be addressed immediately, consistent with guidance issued by NYS DEC.

# Storm Response Reporting

A site inspection report will be submitted to OER at the completion of site inspection. An inspection report established by OER is available on OER’s website (www.nyc.gov/oer) and will be used for this purpose. Site conditions will be compared to the inventory of site conditions and material performed prior to the storm event and significant differences will be noted. The site inspection report will be sent to the OER project manager and will include the site name, address, tax block and lot, site primary and alternate contact name and phone number. Damage and soil release assessment will include: whether the project had stockpiles; whether stockpiles were damaged; photographs of damage and notice of plan for repair; report of whether soil from the site was dislocated and whether any of the soil left the site; estimates of the volume of soil that left the site, nature of impact, and photographs; description of erosion damage; description of equipment damage; description of damage to the remedial program or the construction program, such as damage to the support of excavation; presence of onsite or offsite exposure pathways caused by the storm; presence of petroleum or other spills and status of spill reporting to NYS DEC; description of corrective actions; schedule for corrective actions. This report should be completed and submitted to the OER project manager with photographs within 24 hours of the time of safe entry to the property after the storm event.

# 5.8	Traffic Control

Drivers of trucks leaving the Site with soil/fill will be instructed to proceed without stopping in the vicinity of the Site to prevent neighborhood impacts. The planned route on local roads for trucks leaving the site is shown on Figure 13.

# 5.9	Demobilization

Demobilization will include:

- As necessary, restoration of temporary access areas and areas that may have been disturbed to accommodate support areas (e.g., staging areas, decontamination areas, storage areas, temporary water management areas, and access area);

- Removal of sediment from erosion control measures and truck wash and disposal of materials in accordance with applicable laws and regulations;

- Equipment decontamination, and;

- General refuse disposal.

Equipment will be decontaminated and demobilized at the completion of all field activities.  Investigation equipment and large equipment (e.g., soil excavators) will be washed at the truck inspection station as necessary. In addition, all investigation and remediation derived waste will be appropriately disposed of.

# 5.10	Reporting and Record Keeping

# Daily Status Reports (DSRs)

Daily reports providing a general summary of activities for each day of active remedial work will be emailed to the OER Project Manager by the end of the following business day.  Those reports will include:

- Project number and statement of the activities and an update of progress made and locations of excavation and other remedial work performed;

- Quantities of material imported and exported from the Site;

- Status of on-Site soil/fill stockpiles;

- A summary of all citizen complaints, with relevant details (basis of complaint; actions taken; etc.);

- A summary of CAMP results noting all excursions. CAMP data may be reported;

- Photograph of notable Site conditions and activities.

The frequency of the reporting period may be revised in consultation with OER project manager based on planned project tasks. Daily email reports are not intended to be the primary mode of communication for notification to OER of emergencies (accidents, spills), requests for changes to the RAWP or other sensitive or time critical information.  However, such information will be included in the daily reports.  Emergency conditions and changes to the RAWP will be communicated directly to the OER project manager by personal communication. Daily reports will be included as an Appendix in the Remedial Action Report.

# Record Keeping and Photo Documentation

Job-site record keeping for all remedial work will be performed.  These records will be maintained on-Site during the project and will be available for inspection by OER staff. Representative photographs will be taken of the Site prior to any remedial activities and during major remedial activities to illustrate remedial program elements and contaminant source areas. Photographs will be submitted at the completion of the project in the RAR in digital format (i.e., jpeg files).

# 5.11	Complaint Management

All complaints from citizens will be promptly reported to OER.  Complaints will be addressed, and outcomes will also be reported to OER in daily reports. Notices to OER will include the nature of the complaint, the party providing the complaint, and the actions taken to resolve any problems.

# 5.12	Deviations from The Remedial Action Work Plan

All changes to the RAWP will be reported to, and approved by, the OER Project Manager and will be documented in daily reports and reported in the Remedial Action Report.  The process to be followed if there are any deviations from the RAWP will include a request for approval for the change from OER noting the following:

- Reasons for deviating from the approved RAWP;

- Effect of the deviations on overall remedy; and

- Determination with basis that the remedial action with the deviation(s) is protective of public health and the environment.

# 6.0	Remedial Action Report

A Remedial Action Report (RAR) will be submitted to OER following implementation of the remedial action defined in this RAWP.  The RAR will document that the remedial work required under this RAWP has been completed and has been performed in compliance with this plan.  The RAR will include:

- Information required by this RAWP;

- Text description with thorough detail of all engineering and institutional controls (if Track 1 remedial action is not achieved)

- As-built drawings for all constructed remedial elements;

- Manifests for al soil or fill disposal;

- Photographic documentation of remedial work performed under this remedy;

- Site Management Plan (if Track 1 remedial action is not achieved);

- Description of any changes in the remedial action from the elements provided in this RAWP and associated design documents;

- Tabular summary of all end point sampling results (including all soil test results from the remedial investigation for soil that will remain on site) and all soil/fill waste characterization results, QA/QC results for end-point sampling, and other sampling and chemical analysis performed as part of the remedial action;

- Test results or other evidence demonstrating that remedial systems are functioning properly;

- Account of the source area locations and characteristics of all soil or fill material removed from the Site including a map showing the location of these excavations and hotspots, tanks or other contaminant source areas;

- Full accounting of the disposal destination of all contaminated material removed from the Site. Documentation associated with disposal of all material will include transportation and disposal records, and letters approving receipt of the material;

- Account of the origin and required chemical quality testing for material imported onto the Site;

- Continue registration of the property with an E-Designation by the NYC Department of Buildings (if Track 1 remedial action is not achieved);

- The RAWP and Remedial Investigation Report will be included as appendices to the RAR;

- Reports and supporting material will be submitted in digital form and final PDF’s will include bookmarks for each appendix.

# Remedial Action Report Certification

I, [name], certify the following:

- I am currently a registered professional engineer licensed by the State of New York.

- I performed professional engineering services and had primary direct responsibility for implementation of the remedial program for the [1049 Washington Avenue, Bronx, New York } site, site number [VCP site number].

- I have reviewed this document, to which my signature and seal are affixed.

- Engineering Controls implemented during this remedial action were designed by me or a person under my direct supervision and achieve the goals established in the Remedial Action Work Plan for this site.

If a Track 1 or a Track 2 Restricted Residential Remedial Action was achieved (without an active SSDS), substitute the following passage for above:

The [ vapor barrier, passive SSDS, composite cover system implemented as part of construction] constructed during this remedial action were designed by me or a person under my direct supervision and achieve the goals established in the Remedial Action Work Plan for this site.

- The Engineering Controls constructed during this remedial action were professionally observed by me or by a person under my direct supervision and (1) are consistent with the Engineering Control design established in the Remedial action Work Plan and (2) are accurately reflected in the text and drawings for as-built design reported in this Remedial Action Report.

- The OER-approved Remedial Action Work Plan dated [date] and Stipulations in a letter dated [date] were implemented and that all requirements in those documents have been substantively complied with. I certify that contaminated soil, fill, liquid or other material from the property was taken to facilities licensed to accept this material in full compliance with applicable laws and regulations.

PE Stamp

Name

PE License Number

Signature

Date

I, [Robert M. Bennett], certify the following:

- I am a Qualified Environmental Professional. I had primary direct responsibility for implementation of the remedial program for the [1049 Washington Avenue, Bronx, NY] site, site number [VCP TBD].

- The OER-approved Remedial Action Work Plan dated [date] and Stipulations in a letter dated [date] were implemented and all requirements in those documents have been substantively complied with. I certify that contaminated soil, fill, liquid or other material from the property was taken to facilities licensed to accept this material in full compliance with applicable laws and regulations.

QEP Name

QEP Signature

Date

# 7.0	Schedule

The table below presents a schedule for the proposed remedial action and reporting.  If the schedule for remediation and development activities changes, it will be updated and submitted to OER.  Currently, a 18-month remediation period is anticipated.

| Schedule Milestone | Weeks from Remedial Action Start | Duration (weeks) |
| --- | --- | --- |
| OER Approval of RAWP | 0 | 2 |
| Fact Sheet 2 announcing start of remedy | 0 | 1 |
| Mobilization | 2 | 2 |
| Remedial Excavation | 2 | 30 |
| Demobilization | 82 | 4 |
| Submit Remedial Action Report | 70 | 4 |

# FIGURES

# APPENDIX 1

# PROPOSED DEVELOPMENT PLANS

(SUBMITTED VIA EPIC)

# APPENDIX 2

# CITIZEN PARTICIPATION PLAN

The NYC Office of Environmental Remediation and W. Residence LLC have established this Citizen Participation Plan because the opportunity for citizen participation is an important component of the NYC Voluntary Cleanup Program.  This Citizen Participation Plan describes how information about the project will be disseminated to the Community during the remedial process.  As part of its obligations under the NYC VCP, W. Residence LLC will maintain a repository for project documents and provide public notice at specified times throughout the remedial program.  This Plan also takes into account potential environmental justice concerns in the community that surrounds the project Site.   Under this Citizen Participation Plan, project documents and work plans are made available to the public in a timely manner.  Public comment on work plans is strongly encouraged during public comment periods.  Work plans are not approved by the NYC Office of Environmental Remediation (OER) until public comment periods have expired and all comments are formally reviewed.  An explanation of cleanup plans in the form of a public meeting or informational session is available upon request to OER’s project manager assigned to this Site, Shaminder Chawla, who can be contacted about these issues or any others questions, comments or concerns that arise during the remedial process at.

Project Contact List:  OER has established a Site Contact List for this project to provide public notices in the form of fact sheets to interested members of the Community.  Communications will include updates on important information relating to the progress of the cleanup program at the Site as well as to request public comments on the cleanup plan.  The Project Contact List includes owners and occupants of adjacent buildings and homes, principal administrators of nearby schools, hospitals and day care centers, the public water supplier that serves the area, established document repositories, the representative Community Board, City Council members, other elected representatives and any local Brownfield Opportunity Area (BOA) grantee organizations. Any member of the public or organization will be added to the Site Contact List on request. A copy of the Site Contact List is maintained by OER’s project manager.  If you would like to be added to the Project Contact List, contact NYC OER at (212) 788-8841 or by email at brownfields@cityhall.nyc.gov.

Repositories:  A document repository is maintained online. Internet access to view OER’s document repositories is available at public libraries. This document repository is intended to house, for community review, all principal documents generated during the cleanup program including Remedial Investigation plans and reports, Remedial Action work plans and reports, and all public notices and fact sheets produced during the lifetime of the remedial project. The library nearest the Site is:

Morrisania Library (NYPL)

610 East 169th Street
Bronx, NY 10456

Telephone Number: 718-589-9268

Hours of Operation:

| Mon | 10:00 AM – 7:00 PM |
| --- | --- |
| Tue | 10:00 AM – 7:00 PM |
| Wed | 10:00 AM – 7:00 PM |
| Thu | 10:00 AM – 7:00 PM |
| Fri | 10:00 AM – 5:00 PM |
| Sat | 10:00 AM – 5:00 PM |
| Sun | Closed |

Digital Documentation:  NYC OER requires the use of digital documents in our repository as a means of minimizing paper use while also increasing convenience in access and ease of use.

Issues of Public Concern: The major issues of concern to the public will be potential impacts of nuisance odors and dust during the disturbance of historic fill soils at the Site. This work will be performed in accordance with procedures which will be specified under a detailed Remedial Program which considers and takes preventive measures for exposures to future residents of the property and those on adjacent properties during construction. Detailed plans to monitor the potential for exposure including a Construction Health and Safety Plan and a Community Air Monitoring Plan are required components of the remedial program.

Implementation of these plans will be under the direct oversight of the New York City Department of Environmental Remediation (NYCOER).

These plans will specify the following worker and community health and safety activities during remedial activity at the Site:

  - On-Site air monitoring for worker protection,

  - Perimeter air monitoring for community protection.

The Health and Safety Plan and the Community Air Monitoring Plan prepared as part of the Remedial Action Work Plan will be available for public review at the document repository.

Public Notice and Public Comment:  Public notice to all members of the Project Contact List is required at three major steps during the performance of the cleanup program (listed below) and at other points that may be required by OER.  Notices will include Fact Sheets with descriptive project summaries, updates on recent and upcoming project activities, repository information, and important phone and email contact information.  All notices will be reviewed and approved by OER prior to distribution and mailed by the Enrollee.  Public comment is solicited in public notices for all work plans developed under the NYC Voluntary Cleanup Program.  Final review of all work plans by OER will consider all public comments.  Approval will not be granted until the public comment period has been completed.

Citizen Participation Milestones:  Public notice and public comment activities occur at several steps during a typical NYC VCP project.  These steps include:

- Public Notice of the availability of the Remedial Investigation Report and Remedial Action Work Plan and a 30-day public comment period on the Remedial Action Work Plan:  Public notice in the form of a Fact Sheet is sent to all parties listed on the Site Contact List announcing the availability of the Remedial Investigation Report and Remedial Action Work Plan and the initiation of a 30-day public comment period on the Remedial Action Work Plan.  The Fact Sheet summarizes the findings of the RIR and provides details of the RAWP.  The public comment period will be extended an additional 15 days upon public request.  A public meeting or informational session will be conducted by OER upon request.

- Public Notice announcing the approval of the RAWP and the start of remediation:  Public notice in the form of a Fact Sheet is sent to all parties listed on the Site Contact List announcing the approval of the RAWP and the start of remediation.

- Public Notice announcing the completion of remediation, designation of Institutional and Engineering Controls and issuance of the Notice of Completion:  Public notice in the form of a Fact Sheet is sent to all parties listed on the Site Contact List announcing the completion of remediation, providing a list of all Institutional and Engineering Controls implemented for to the Site and announcing the issuance of the Notice of Completion.

# APPENDIX 3

# SUSTAINABILITY STATEMENT

This Sustainability Statement documents sustainable activities and green remediation efforts planned under this remedial action.

Reuse of Clean, Recyclable Materials. :  Reuse of clean, locally derived recyclable materials reduce consumption of non-renewable virgin resources and can provide energy savings and greenhouse gas reduction. This project intends to use recycled concrete aggregate wherever possible in grading and backfilling the Site. Clean, non-virgin materials, such as clean sand/soil meeting Track 1 Unrestricted Use SCOs from the Clean Soil Bank and/or other distributors may be imported as needed.   However, the redevelopment plan does not account for any need of such materials.  In the event that such materials become needed for backfilling over excavated areas and/or around foundation elements, the total amount imported will quantified and reported in the RAR. Records/tracking documents and daily reports will be collected to document the import details.

Reduced Consumption of Virgin and Non-Renewable Resources. Reduced consumption of virgin and non-renewable resources lowers the overall environmental impact of the project on the region by conserving these resources.  The project will reduce the consumption of virgin materials by substituting recycled concrete aggregate for mined gravel and/or sand backfill whenever possible.

An estimate of the quantity 650 cubic yards / 950 metric tons of virgin-mined ¾-inch bluestone will be used under this plan as part of the passive SSDS.  The total imported quantity will be quantified and reported in the RAR.

Reduced Energy Consumption and Promotion of Greater Energy Efficiency:  Reduced energy consumption lowers greenhouse gas emissions, improves local air quality, lessens in-city power generation requirements, can lower traffic congestion, and provides substantial cost savings.

Recycled concrete materials and other backfill materials will be locally sourced reducing the energy consumption associated with transporting these materials to the Site. Best efforts will be made to quantify energy efficiencies achieved during the remediation and will be reported in the Remedial Action Report (RAR).  Where energy savings cannot be easily quantified, a gross indicator of the amount of energy saved or the means by which energy savings were achieved will be reported.

Conversion to Clean Fuels:  Use of clean fuel improves NYC’s air quality by reducing harmful emissions. Natural gas will be utilized for fuel in the new building.  An estimate of the volume of clean fuels used during remedial activities will be quantified and reported in the RAR.

Recontamination Control:  Recontamination after cleanup and redevelopment is completed undermines the value of work performed, may result in a property that is less protective of public health or the environment, and may necessitate additional cleanup work later or impede future redevelopment.  Recontamination can arise from future releases that occur within the property or by influx of contamination from off-Site.

An estimate of the area of the Site that utilizes recontamination controls under this plan will be reported in the RAR in square feet.

Stormwater Retention:  Stormwater retention improves water quality by lowering the rate of combined stormwater and sewer discharges to NYC’s sewage treatment plants during periods of precipitation and reduces the volume of untreated influent to local surface waters.

An estimate of the enhanced stormwater retention capability of the redevelopment project will be included in the RAR.

Linkage with Green Building:  Green buildings provide a multitude of benefits to the city across a broad range of areas, such as reduction of energy consumption, conservation of resources, and reduction in toxic materials use.

Paperless Voluntary Cleanup Program:  W. Residence LLC is participating in OER’s Paperless Voluntary Cleanup Program.  Under this program, submission of electronic documents will replace submission of hard copies for the review of project documents, communications and milestone reports.

Low-Energy Project Management Program:  W. Residence LLC is participating in OER’s low-energy project management program.  Under this program, whenever possible, meetings are held using remote communication technologies, such as videoconferencing and teleconferencing to reduce energy consumption and traffic congestion associated with personal transportation.

# APPENDIX 4

# SOIL/MATERIALS MANAGEMENT PLAN

# 1.1 Soil Screening Methods

Visual, olfactory and PID soil screening and assessment will be performed under the supervision of a Qualified Environmental Professional and will be reported in the final remedial report.  Soil screening will be performed during invasive work performed during the remedy and development phases prior to issuance of final signoff by OER.

# 1.2 Stockpile Methods

Excavated soil from suspected areas of contamination (e.g., hot spots, USTs, drains, etc.) will be stockpiled separately and will be segregated from clean soil and construction materials. Stockpiles will be used only when necessary and will be removed as soon as practicable. While stockpiles are in place, they will be inspected daily, and before and after every storm event.  Results of inspections will be recorded in a logbook and maintained at the Site and available for inspection by OER. Excavated soils will be stockpiled on, at minimum, double layers of 8-mil minimum sheeting, will be kept covered at all times with appropriately anchored plastic tarps, and will be routinely inspected.  Broken or ripped tarps will be promptly replaced.

All stockpile activities will be compliant with applicable laws and regulations. Soil stockpile areas will be appropriately graded to control run-off in accordance with applicable laws and regulations. Stockpiles of excavated soil and other materials shall be located at least of 50 feet from the property boundaries, where possible. Hay bales or equivalent will surround soil stockpiles except for areas where access by equipment is required. Silt fencing and hay bales will be used as needed near catch basins, surface waters and other discharge points.

# 1.3 Characterization of Excavated Materials

Soil/fill or other excavated media that is transported off-Site for disposal will be sampled in a manner required by the receiving facility, and in compliance with applicable laws and regulations. Soils proposed for reuse on-Site will be managed as defined in this plan.

# 1.4 Materials Excavation, Load-Out, and Departure

The PE/QEP overseeing the remedial action will:

- oversee remedial work and the excavation and load-out of excavated material;

- ensure that there is a party responsible for the safe execution of invasive and other work performed under this work plan;

- ensure that Site development activities and development-related grading cuts will not interfere with, or otherwise impair or compromise the remedial activities proposed in this RAWP;

- ensure that the presence of utilities and easements on the Site has been investigated and that any identified risks from work proposed under this plan are properly addressed by appropriate parties;

- ensure that all loaded outbound trucks are inspected and cleaned if necessary, before leaving the Site;

- ensure that all egress points for truck and equipment transport from the Site will be kept clean of Site-derived materials during Site remediation.

Locations where vehicles exit the Site shall be inspected daily for evidence of soil tracking off premises. Cleaning of the adjacent streets will be performed as needed to maintain a clean condition with respect to Site-derived materials.

Open and uncontrolled mechanical processing of historical fill and contaminated soil on-Site will not be performed without prior OER approval.

# 1.5 Off-Site Materials Transport

Loaded vehicles leaving the Site will comply with all applicable materials transportation requirements (including appropriate covering, manifests, and placards) in accordance with applicable laws and regulations, including use of licensed haulers in accordance with 6 NYCRR Part 364. If loads contain wet material capable of causing leakage from trucks, truck liners will be used. Queuing of trucks will be performed on-Site, when possible, in order to minimize off Site disturbance.  Off-Site queuing will be minimized.

Outbound truck transport routes are described in the remedial report. This routing takes into account the following factors: (a) limiting transport through residential areas and past sensitive sites; (b) use of mapped truck routes; (c) minimizing off-Site queuing of trucks entering the facility; (d) limiting total distance to major highways; (e) promoting safety in access to highways; and (f) overall safety in transport. To the extent possible, all trucks loaded with Site materials will travel from the Site using these truck routes. Trucks will not stop or idle in the neighborhood after leaving the project Site.

# 1.6 Materials Disposal Off-Site

The following documentation will be established and reported by the PE/QEP for each disposal destination used in this project to document that the disposal of regulated material exported from the Site conforms with applicable laws and regulations: (1) a letter from the PE/QEP or Enrollee to each disposal facility describing the material to be disposed and requesting written acceptance of the material.  This letter will state that material to be disposed of is regulated material generated at an environmental remediation Site in New York City under a governmental remediation program.  The letter will provide the project identity and the name and phone number of the PE/QEP or Enrollee.  The letter will include as an attachment a summary of all chemical data for the material being transported; and (2) a letter from each disposal facility stating it is in receipt of the correspondence (1, above) and is approved to accept the material.  These documents will be included in the final remedial report.

The Remedial Action Report will include an itemized account of the destination of all material removed from the Site during this remedial action.  Documentation associated with disposal of all material will include records and approvals for receipt of the material.  This information will be presented in the final remedial report.

All impacted soil/fill or other waste excavated and removed from the Site will be managed as regulated material and will be disposed of in accordance with applicable laws and regulations.  Historic fill and contaminated soil taken off-Site will be handled as solid waste and will not be disposed at a Part 360-16 Registration Facility (also known as a Soil Recycling Facility).

Waste characterization will be performed for off-Site disposal in a manner required by the receiving facility and in conformance with its applicable permits.  Waste characterization sampling and analytical methods, sampling frequency, analytical results and QA/QC will be reported in the final remedial report. A manifest system for off-Site transportation of exported materials will be employed.  Manifest information will be reported in the final remedial report. Hazardous waste derived from on-Site will be stored, transported, and disposed of in compliance with applicable laws and regulations.

If disposal of soil/fill from this Site is proposed for unregulated disposal (i.e., clean soil removed for development purposes), including transport to a Part 360-16 Registration Facility, a formal request will be made for approval by OER with an associated plan compliant with 6NYCRR Part 360-16. This request and plan will include the location, volume and a description of the material to be recycled, including verification that the material is not impacted by site uses and that the material complies with receipt requirements for recycling under 6NYCRR Part 360.  This material will be appropriately handled on-Site to prevent mixing with impacted material.

# 1.7 Materials Reuse On-Site

Soil and fill that is derived from the property that meets the Soil Cleanup Objectives (SCOs) established in this plan may be reused on-Site. The SCOs for on-Site reuse are listed in Section 4.2 of this cleanup plan. ‘Reuse On-Site’ means material that is excavated during the remedy or development, does not leave the property, and is relocated within the same property and on land with comparable levels of contaminants in soil/fill material, compliant with applicable laws and regulations, and addressed pursuant to the NYC VCP agreement subject to Engineering and Institutional Controls. The PE/QEP will ensure that reused materials are segregated from other materials to be exported from the Site and that procedures defined for material reuse in this remedial plan are followed.

Organic matter (wood, roots, stumps, etc.) or other waste derived from clearing and grubbing of the Site will not be buried on-Site. Soil or fill excavated from the site for grading or other purposes will not be reused within a covered soil layer or within landscaping berms.

# 1.8 Demarcation

After completion of hotspot removal and any other invasive remedial activities, and prior to backfilling, the top of the residual soil/fill will be defined by one of three methods: (1) placement of a demarcation layer. The demarcation layer will consist of geosynthetic fencing or equivalent material to be placed on the surface of residual soil/fill to provide an observable reference layer.  A description or map of the approximate depth of the demarcation layer will be provided in the SMP; or (2) a land survey of the top elevation of residual soil/fill before the placement of cover soils, pavement and associated sub-soils, or other materials or structures or, (3) all materials beneath the approved cover will be considered impacted and subject to site management after the remedy is complete. Demarcation may be established by one or any combination of these three methods. As appropriate, a map showing the method of demarcation for the Site and all associated documentation will be presented in the RAR.

This demarcation will constitute the top of the site management horizon. Materials within this horizon require adherence to special conditions during future invasive activities as defined in the Site Management Plan.

# 1.9 Import of Backfill Soil from Off-Site Sources

All imported soil will meet OER-approved backfill and cover soil quality objectives for this Site. Imported soils will not exceed groundwater protection standards established in Part 375. Imported soil for Track 1 remedial action projects will not exceed Track 1 SCO’s.

A process will be established to evaluate sources of backfill and cover soil to be imported to the Site, and will include an examination of source location, current and historical use(s), and any applicable documentation. Material from industrial sites, spill sites, environmental remediation sites or other potentially contaminated sites will not be imported to the Site.

The following potential sources may be used pending attainment of backfill and cover soil quality objectives:

- Clean soil from construction projects at non-industrial sites in compliance with applicable laws and regulations;

- Clean soil from roadway or other transportation-related projects in compliance with applicable laws and regulations;

- Clean recycled concrete aggregate (RCA) from facilities permitted or registered by the regulations of NYS DEC.

All materials received for import to the Site will be approved by a PE/QEP and will be in compliance with provisions in this remedial plan.  The final remedial report will report the source of the fill, evidence that an inspection was performed on the source, chemical sampling results, frequency of testing, and a Site map indicating the locations where backfill or soil cover was placed.

Source Screening and Testing

Inspection of imported fill material will include visual, olfactory and PID screening for evidence of contamination. Materials imported to the Site will be subject to inspection, as follows:

- Trucks with imported fill material will be in compliance with applicable laws and regulations and will enter the Site at designated locations;

- The PE/QEP is responsible to ensure that every truck load of imported material is inspected for evidence of contamination; and

- Fill material will be free of solid waste including pavement materials, debris, stumps, roots, and other organic matter, as well as ashes, oil, perishables or foreign matter.

Composite samples of imported material will be taken at a minimum frequency of one sample for every 500 cubic yards of material. Once it is determined that the fill material meets imported backfill or cover soil chemical requirements and is non-hazardous, and lacks petroleum contamination, the material will be loaded onto trucks for delivery to the Site.

Recycled concrete aggregate (RCA) will be imported from facilities permitted or registered by NYSDEC.  Facilities will be identified in the final remedial report.  A PE/QEP is responsible for ensuring that the facility is compliant with 6NYCRR Part 360 registration and permitting requirements for the period of acquisition of RCA.  RCA imported from compliant facilities will not require additional testing, unless required by NYSDEC under its terms for operation of the facility.  RCA imported to the Site must be derived from recognizable and uncontaminated concrete. RCA material is not acceptable for and will not be used as cover material.

# 1.10 Fluids Management

All liquids to be removed from the Site, including dewatering fluids, will be handled, transported and disposed of in accordance with applicable laws and regulations.  Liquids discharged into the New York City sewer system will receive prior approval by the New York City Department of Environmental Protection (NYC DEP). The NYC DEP regulates discharges to the New York City sewers under Title 15, Rules of the City of New York Chapter 19.  Discharge to the New York City sewer system will require an authorization and sampling data demonstrating that the groundwater meets the City’s discharge criteria. The dewatering fluid will be pretreated as necessary to meet the NYC DEP discharge criteria.  If discharge to the city sewer system is not appropriate, the dewatering fluids will be managed by transportation and disposal at an off-Site treatment facility.

Discharge of water generated during remedial construction to surface waters (i.e., a stream or river) is prohibited without an SPDES permit issued by New York State Department of Environmental Conservation.

# 1.11 Stormwater Pollution Prevention

Applicable laws and regulations pertaining to stormwater pollution prevention will be addressed during the remedial program. Erosion and sediment control measures identified in this remedial plan (silt fences and barriers, and hay bale checks) will be installed around the entire perimeter of the remedial construction area and inspected once a week and after every storm event to ensure that they are operating appropriately. Discharge locations will be inspected to determine whether erosion control measures are effective in preventing significant impacts to receptors. Results of inspections will be recorded in a logbook and maintained at the Site and available for inspection by OER.  All necessary repairs shall be made immediately. Accumulated sediments will be removed as required to keep the barrier and hay bale check functional. Undercutting or erosion of the silt fence toe anchor will be repaired immediately with appropriate backfill materials. The manufacturer’s recommendations will be followed for replacing silt fencing damaged due to weathering.

# 1.12 Contingency Plan for Unknown Contamination Sources

This contingency plan is developed for the remedial construction to address the discovery of unknown structures or contaminated media during excavation. Identification of unknown contamination source areas during invasive Site work will be promptly communicated to OER’s Project Manager. Petroleum spills will be reported to the NYS DEC Spill Hotline. These findings will be included in the daily report. If previously unidentified contaminant sources are found during on-Site remedial excavation or development-related excavation, sampling will be performed on contaminated source material and surrounding soils and reported to OER.  Chemical analytical testing will be performed for TAL metals, TCL volatiles and semi-volatiles, TCL pesticides and PCBs, as appropriate.

# 1.13 Odor, Dust, and Nuisance Control

# Odor Control

All necessary means will be employed to prevent on- and off-Site odor nuisances.  At a minimum, procedures will include: (a) limiting the area of open excavations; (b) shrouding open excavations with tarps and other covers; and (c) use of foams to cover exposed odorous soils. If odors develop and cannot otherwise be controlled, additional means to eliminate odor nuisances will include: (d) direct load-out of soils to trucks for off-Site disposal; and (e) use of chemical odorants in spray or misting systems.

This odor control plan is capable of controlling emissions of nuisance odors.  If nuisance odors are identified, work will be halted, and the source of odors will be identified and corrected.  Work will not resume until all nuisance odors have been abated.  OER will be notified of all odor complaint events.  Implementation of all odor controls, including halt of work, will be the responsibility of the PE/QEP’s certifying this remedial plan.

# Dust Control

Dust management during invasive on-Site work will include, at a minimum:

- Use of a dedicated water spray methodology for roads, excavation areas and stockpiles.

- Use of properly anchored tarps to cover stockpiles.

- Exercise extra care during dry and high-wind periods.

- Use of gravel or recycled concrete aggregate on egress and other roadways to provide a clean and dust-free road surface.

This dust control plan is capable of controlling emissions of dust.  If nuisance dust emissions are identified, work will be halted and the source of dust will be identified and corrected.  Work will not resume until all nuisance dust emissions have been abated.  OER will be notified of all dust complaint events.  Implementation of all dust controls, including halt of work, will be the responsibility of the PE/QEP’s responsible for certifying this remedial plan.

# Other Nuisances

Noise control will be exercised during the remedial program. All remedial work will conform, at a minimum, to NYC noise control standards.

Rodent control will be provided during Site clearing and grubbing and during the remedial program, as necessary, to prevent nuisances.

# APPENDIX 5

# CONSTRUCTION HEALTH AND SAFETY PLAN

# APPENDIX 6

VAPOR BARRIER SPECIFICATIONS & INSTALLATION INSTRUCTIONS

# APPENDIX 7

PASSIVE SUB-SLAB VENTING SYSTEM SPECIFICATIONS

PE Stamp